Ambient Noise in the Arctic, 1986  (ANITA-86)

Current Meter Record

Two moorings were deployed in eastern Amundsen Gulf by FSRG personnel on

April 2, 1986.  Both moorings carried a WOTAN ambient noise recording

instrument, and one on the moorings (at WOTAN-2) carried a recording current at

a depth of 120 m.

Note that in the publication by Waddell and Farmer (JGR, 93, 2333-2342), the

location of the current meter is WRONGLY specified as mooring WOTAN-1.

WOTAN-2	69 31.6' N	117 25.0' W	RCM4 4182	220m	120m

Start  2 April,     1986 at 2000 GMT

End   25 September, 1986 at 0800 GMT

The RCM was deployed on April 2, 1986 and recovered on 25 September, 1986,

by Dave English on the CCGS Martha Black.

Note that the direction was not calibrated using direction corrections, and

that the coefficient used was 360/1023 instead of 360/1024.

C4182-86.RAW	TC	P	CR	T	D	U

C4182-86.DAT	#	T	CR	P	D	U	N/S E/W S RHO

-------------------------------------------------------

** Original Header of the calibrated data file from AYPLEE **

-------------------------------------------------------

86  WOTA6931.36N11725.00W8604022000GMT  0IMP86       10JTCPBVNESG   120 42130.02

AANDERAA CURRENT METER: RCM4

IMP 86                   WOTAN 2   C4182/IMP           SUBSURFACE

60.00000      0.0000000E+00        1986           0           0

86           4           2          20  0.0000000E+00        2228

86           9          25           8  0.0000000E+00        6440

T:   -0.26900E+01    0.56312E-02   -0.56249E-07    0.22660E-10    0.00000E+00

R:    0.10000E+01    0.50857E+00    0.18084E-03    0.00000E+00    0.00000E+00

P:   -0.89000E+01    0.15480E+00    0.00000E+00    0.00000E+00    0.00000E+00

D:    0.46000E+02    0.00000E+00    0.35190E+00    0.00000E+00    0.00000E+00

S:    0.22000E+01    0.89330E-01    0.00000E+00    0.00000E+00    0.00000E+00

(I6,F7.2,F7.4,F7.1,4F6.1,2F6.2)

@COPYRIGHT, 1991.  GOVERNMENT OF CANADA

VARIABLE 1=TIME, TO GET JULIAN DAY FROM JAN 1, 86/ # OF RECORDS PER DAY

2=TEMPERATURE (C)

3=CONDUCTIVITY

4=PRESSURE (DBARS)

5=CURRENT DIRECTION (DEG TRUE NORTH)

6=CURRENT SPEED (CM/S)

7=NS COMPONENTS (CM/S)

8=EW      "

9=SALINITY (PPT)

10=SIGMA-T

FOR INFORMATION, PLEASE CONTACT  ANDREW Y P LEE

P.O. BOX 6000, SIDNEY, BC, CANADA   V8L 4B2  (604)363-6340

In order to re-calibrate the data in the absence of the original raw data, the

available calibrated data file was uncalibrated using the Fortran routine

UNDO_CAL.FOR appended below.  The file was recalibrated using standard MLH

procedures.

Humfrey Melling

May 22, 1991

--------------------------------------------------------------------------------

Routine UNDO_CAL.FOR to uncalibrate the file U00743_034.DAT, obtained from

A.Y.P. Lee

OPTIONS /EXTEND_SOURCE

C	Decodes erroneously calibrated Aanderaa Data

C

IMPLICIT NONE

REAL temperature,ratio,pressure,direction,speed,test_t,dif_t

INTEGER NT,NR,NP,ND,NS,COUNT,NREC

OPEN (UNIT=7,FILE='U00743_034.DAT',ACCESS='SEQUENTIAL',STATUS='OLD',

1 FORM = 'FORMATTED')

OPEN (UNIT=8,FILE='RCM4_4182.DATA_RAW',ACCESS='SEQUENTIAL',STATUS='NEW',

1 FORM = 'FORMATTED')

OPEN (UNIT=9,FILE='RCM4_4182.DATA_OLD',ACCESS='SEQUENTIAL',STATUS='NEW',

1 FORM = 'FORMATTED')

DO COUNT=1,25

READ (7,*)

ENDDO

COUNT=0

1	READ (7,100,END=999)NREC,temperature,ratio,pressure,direction,speed

100	FORMAT (6X,I6,1X,F10.6,5X,F10.7,5X,F10.4,5X,F10.4/1X,F10.6)

COUNT=COUNT+1

NR=JNINT((ratio-0.50857)/0.00018084)

NP=JNINT((pressure+8.9)/0.1548)

NS=JNINT((speed-2.2)/0.08933)

ND=JNINT(AMOD(direction-46.0+360.0,360.)/0.3519)

NT=10

dif_t=100.

20	test_t=-2.69+NT*(0.0056312+NT*(-5.6249e-8+NT*(2.266e-11)))

if (ABS(temperature-test_t).le.dif_t) then

NT=NT+1

dif_t=ABS(temperature-test_t)

go to 20

else

NT=NT-1

endif

WRITE (8,101)NREC,NT,NR,NP,ND,NS

101	FORMAT (6I6)

WRITE (9,102)FLOAT(NREC),temperature,ratio,pressure,direction,speed

102	FORMAT (1X,F6.0,1X,F7.3,1X,F9.6,1X,F8.2,1X,F8.2,1X,F8.2)

GO TO 1

999	WRITE (6,*) ' End of file reached.  Count is ... ',COUNT

CLOSE (7)

CLOSE (8)

CLOSE (9)

END