Current meter conversion notes

May 11, 1998.

Mission  : Deployment

Recovery   (84-14, 84-15)

Date     : June 19, 1984 to Nov 26, 1984.

Project  : CCEXP -- Coastal Current Experiment

Station  : J2S, T2S, T3S, VCB, VE1, VE2, VJ1, VJ2, VJ3, VJ4, VN1,

VT2, VT3, VT4, VT5, VT6, W01 and W02.

Comments:

There were 45 files converted from CHS format.

--------------------------------------------------------------------------------

** J2S **

Juan de Fuca

J2S_0005.CUR -- 5m

WARNING: Shallow VMCM's under surface moorings may be inaccurate at higher

frequencies. Flow component at 60 deg (peak in hist) may be swell induced.

No documentation as to processing; assume on time.

Recovered on cruise IOSOP84-15.

EG&G SeaLink Vector-Measuring Current Meter (VMCM), on surface mooring.

Current meters are owned, and data were processed by Univ. of Washington.

Velocities are vector averages, even though files are not "drav".

Chan #4 not assigned.  No tape #; assumed #1.  Full s/n 100208.

--removed No Sensor channel

J2S_0010.CUR -- 10m

WARNING: Shallow VMCM's under surface moorings may be inaccurate at higher

frequencies. Flow component at 60 degrees (peak in hist) may be swell induced

Recovered on cruise IOSOP84-15.

No documentation as to processing; assume on time.

EG&G SeaLink Vector-Measuring Current Meter (VMCM), on surface mooring.

Current meters are owned, and data were processed by Univ. of Washington.

Velocities are vector averages, even though files are not "drav".

Chan #4 not assigned.  No tape #; assumed #1.  Full s/n 100308.

--removed No Sensor channel

--------------------------------------------------------------------------------

** T2S and T3S **

Tofino

T2S_0005.CUR -- 5m

WARNING: Shallow VMCM's under surface moorings may be inaccurate at higher

frequencies.

No documentation as to timing.        Trimmed to #10927 as mooring appears to

have broken loose here.   Recovered on cruise IOSOP84-14.

EG&G SeaLink Vector-Measuring Current Meter (VMCM), on surface mooring.

Current meters are owned, and data were processed by Univ. of Washington.

Velocities are vector averages, even though files are not "drav".

Chan #4 not assigned.  No tape #; assumed #1.

--removed NO Sensor channel.

T2S_0010.CUR -- 10m

WARNING: Shallow VMCM's under surface moorings may be inaccurate at higher

frequencies.

Recovered IOSOP84-14.                 Trimmed to #10927 as mooring appears to

have broken loose here. No info on processing; assumed on time.

EG&G SeaLink Vector-Measuring Current Meter (VMCM), on surface mooring.

Current meters are owned, and data were processed by Univ. of Washington.

Velocities are vector averages, even though files are not "drav".

Chan #4 not assigned.  No tape #; assumed #1.

--removed No Sensor channel

T3S_0005.CUR -- 5m

WARNING: Shallow VMCM's under surface moorings may be inaccurate at higher

frequencies.  No documentation on timing; assumed on time.

Recovered on IOS cruise IOSOP84-14.

EG&G SeaLink Vector-Measuring Current Meter (VMCM), on surface mooring.

Current meters are owned, and data were processed by Univ. of Washington.

Velocities are vector averages, even though files are not "drav".

Chan #4 not assigned.  No tape #; assumed #1.

--removed No Sensor channel

T3S_0010.CUR -- 10m

WARNING: Shallow VMCMs under surface flotation may be inaccurate at higher

frequencies.  No documentation on timing; assumed on time.

Recovered on IOS cruise IOSOP84-14.

EG&G SeaLink Vector-Measuring Current Meter (VMCM), on surface mooring.

Current meters are owned, and data were processed by Univ. of Washington.

Velocities are vector averages, even though files are not "drav".

Chan #4 not assigned.  No tape #; assumed #1.

--removed No Sensor channel

T3S_0020.CUR -- 20m

WARNING: Shallow VMCMs under surface flotation may be inaccurate at higher

frequencies.  No documentation as to timing; assumed on time.

Recovered during IOS cruise IOSOP84-14.

Range of pressure sensor not available. Res appears to be 0.3 db.

EG&G SeaLink Vector-Measuring Current Meter (VMCM), on surface mooring.

Current meters are owned, and data were processed by Univ. of Washington.

Velocities are vector averages, even though files are not "drav".

Chan #4 not assigned.  No tape #; assumed #1.

--removed No Sensor channel

--------------------------------------------------------------------------------

** VCB **

Cape Beale(nearshore)

VCBA0030.CUR -- 30m. First deployment.

RCM4 1510/11 hit and recovered early by fishermen

Conductivity coeffs wrong, so cond/sal no good.

Ch. 4 is uncalibrated (Pr.?) channel.

Instrument depth is nominal.

Recovered on cruise IOSOP84-14.

--removed Unknown channel

VCBB0030.CUR -- 30m. Second deployment.

RCM4 1510/12  second deployment                                 Conductivity

coeffs wrong, so cond/sal no good.

Instrument depth is nominal.

Channel 4 was uappears to be an uncalibrated pr channel.

Deployed by the "Arrawac Freighter", since renamed the "Beatrice".

Recovered on cruise IOSOP84-14.

--removed Unknown channel

VCBA0050.CUR -- 50m. First deployment.

RCM4 1565/9  hit and recovered early by fisherman.

Assumed on time till then.

Instrument depth is nominal.

Channel 4 was unassigned.

Salinities calculated by the Perkin & Walker formula

--removed No Sensor channel

VCBB0050.CUR -- 50m. Second deployment.

RCM4 1565/10  second deployment

Deployed by the "Arrawac Freighter", since renamed the "Beatrice".

SY    4409LR       0SR       0AB      5

6BE       0PE       0

Salinities calculated by the Perkin & Walker formula

Instrument depth is nominal.

Channel 4 was unassigned.

Recovered on cruise IOSOP84-14.

--removed No Sensor channel

--------------------------------------------------------------------------------

** VE1 and VE2 **

Estevan Point, VE1(nearshore), VE2(offshore)

VE1_0030.CUR -- 30m

RCM5 5213/6  low battery on recovery, no last record            Direction bad

after 8700, temp,cond & sal bad after 9115.  cond/sal low 3786-3829 & 6011-6

036.                                  SY    9508LR       0SR       0AB

0BE       1PE       0

Salinities calculated by the Perkin & Walker formula.

Instrument depth is nominal.

Channel 4 appears to be (uncalibrated) pressure.

Recovered on cruise IOSOP84-14.

--removed Unknown channel

VE1_0050.CUR -- 50m

RCM4 3440/7 translated using CMT77 - tape premagnetized         Coarse temper

ature calibration is approximate. Approximately    2500 spikes interpolated.

Instrument depth is nominal.

Channel  3 was unassigned.  Ch 4 appears to be uncalibrated pressure ch.

Recovered on cruise IOSOP84-14.

--removed No Sensor and Unknown channels

VE2_0030.CUR -- 30m

RCM4 2491/5 Leaked through pressure sensor, tape head broken off on recovery.

1442 records short; assumed to be on time.

Coarse temperature calibration is approximate.

Instrument depth is nominal.

Channel 3 was unassigned; ch 4 appears to be (uncalibrated) pressure ch.

Recovered on cruise IOSOP84-14.

--removed No Sensor and Unknown channels

VE2_0050.CUR -- 50m

RCM4 1297/10                                                    Coarse temper

ature calibration is approximate.

Instrument depth is nominal.

Channels 3 & 4 were not assigned (0's and 1023's respectively.

Recovered on cruise IOSOP84-14.

--removed two No Sensor channels

--------------------------------------------------------------------------------

** VJ1, VJ2, VJ3 and VJ4 **

Juan de Fuca

VJ1_0030.CUR -- 30m

RCM4 1511/9 Mooring hit by fishing gear July 30-31, recovered   top float and

1511, remainder of mooring located Sept 3         no stop time available, as

sume instrument on time!  Strange     temperature/sal drops - appear real so

left in.

Salinities calculated by the Perkin & Walker formula

Instrument depth is nominal.

Channel 4 is an uncalibrated channel (poss pr?).

Current speeds up to 1 m/s may have produced large mooring tilts.

--Removed Unknown channel

VJ1_0050.CUR -- 50m

RCM5 4922/5 mooring hit by fishing gear July 30-31, top float and instrument

recovered.  1 record short on translation, assume on time.

6254:SY      3:LR      1:SR      0:A

Instrument belongs to U. Washington.

Salinities calculated by the Perkin & Walker formula.

Instrument depth is nominal.

Channel 4 was unassigned.

Recovered on cruise IOSOP84-14.

--Removed No Sensor channel

VJ1_0100.CUR -- 100m

RCM4 3387/1 Mooring hit by fishing gear July 30-31, top float  and instrument

recovered.  Remainder of mooring located Sept 3 and returned by fisherman.

Sep 12.  Exp temperature: 193 records were padded.

Salinities calculated by the Perkin & Walker formula.

Instrument depth is nominal.

VJ2_0030.CUR -- 30m

RCM4 6561/1   Large variations in Cond/Sal due to large depth excursions of

instrument; Pressure ranged from 29 to 118 db.

Instrument depth given as nominal 30m, average pressure was 41 db.

Pressure range not given; appears to be 500 psi.

Salinities computed using Perkin & Walker formula.

Some confusion as to water depth; 5m of Nilspin was added above the anchor.

Recovered on cruise IOSOP84-14.

VJ2_0050.CUR -- 50m

RCM4 4553/7  data deteriorates after seq# 1117, with a lot of  work some data

could be recovered at end of file, but timing is questionable.

Record was clipped; is only 14 days long.

RCM is owned by U. Washington.

Salinities calculated by the Perkin & Walker formula are too high; mean=36.2

Channel 4 is an uncalibrated channel.

Large excursions recorded on upper RCM (30m); expect same here.

Recovered on cruise IOSOP84-14.

--removed Unknown channel

VJ2_0125.CUR -- 125m

RCM4 7265/1

RCM is owned by U. Wash.

Large vertical excursions were recorded on the RCM at 30 m.

Instrument depth is nominal.

Channel 3 is an uncalibrated channel; ch 4 was not assigned (1023's).

RCM is owned by U. Washington.

Recovered on cruise IOSOP84-14.

--removed No Sensor and Unknown channels

VJ2_0175.CUR -- 175m

RCM4 3494/1 tape premagnetized - poor translation. Deleted 68 records to corr

ect time, last 3 recs not recorded. Time in and out of water now checks. Many

blocks of bad data in ch 2,3,4,&5.   Ch 2,3&4 padded, ch5 (dir) was interpola

ted by eye. Instrument depth is nominal.  Hist direction peak at 60deg looks

strange (swell induced?). Salinities calc by the Perkin & Walker formula.

RCM is owned by IOS (Ocean Physics/Fr. Seas).

Large vertical excursions were recorded on the upper (30m) RCM.

Recovered on cruise IOSOP84-14.

VJ3_0030.CUR -- 30m

RCM4-L 6527/1

Instrument depth is nominal.

WARNING! Large vertical excursions observed (Pr. sensor ranged from 40 to 106

db)

Salinities calculated by the Perkin & Walker formula.

Pressure range was not given; appears to be 500 psi.

Recovered on cruise IOSOP84-14.

VJ3_0050.CUR -- 50m

RCM4 5099/6 Poor translation after seq#7600.  Trimmed file to   seq#7611, del

eted 5 records before 7611.  Assume on time!

Instrument depth is nominal. WARNING! Large vertical excursions were recorded

on instrument immediately above (30 m).

Salinities calculated by the Perkin & Walker formula

Ch 4 is an uncalibrated channel.

RCM is owned by U. Washington.

Recovered on cruise IOSOP84-14.

--removed Unknown channel

VJ3_0100.CUR -- 100m

RCM4 5097/5 rotor damaged on recovery. Multiple synch pulses on recovery. Tra

nslation poor after seq#5760.  Trim to seq#5760.   Assume on time after delet

ing 2 recs. RCM is owned by U. Washington.

Instrument depth is nominal.  Large vert. exc. recorded at upper RCM (30m)

Salinities calculated by the Perkin & Walker formula

Channel 4 is uncalibrated channel, under-ranging.

Recovered on cruise IOSOP84-14.

--removed Unknown channel

VJ4_0030.CUR -- 30m

RCM4 1511/10  no response to release at 1214 26/11/84.  Found   adrift by fis

herman at Edis Hook (123 27,48 9) 28/11/84.  No endtime as batteries dead.

Instrument depth is nominal. Large vertical excursions are possible.

Channel 4 appears to be an uncalibrated pressure channel.

Salinities calculated by the Perkin & Walker formula

Deployed by "Arrawac Freighter", now the "Beatrice".

--removed Unknown channel

--------------------------------------------------------------------------------

** VN1 and VN2 **

Cape Beale (offshore)

VN1_0030.CUR -- 30m

RCM4 7463/1  tape was premagnetized.

RCM is owned by UBC.  3 records were deleted to correct time.

Significant vertical excursions could have occurred at higher speeds.

Instrument depth is nominal.

Channel 3 was uncalibrated; ch 4 was unassigned (1023's).

Recovered on cruise IOSOP84-14.

--removed Unknown and No Sensor channels

VN1_0050.CUR -- 50m

RCM4 7462/1 tape slightly premagnetized, fair translation.

Instrument is owned by UBC.

Depth is nominal.

Channel 3 is uncalibrated; ch 4 is unassigned (1023's).

Apparently the record was "on time".

Recovered on cruise IOSOP84-14.

--removed Unknown and No Sensor channels

VT2_0045.CUR -- 45m

RCM4 7338/1    Expanded temperature (Ch 4) overranged.

Instrument depth is nominal.

100 C/Sal values are padded.

Salinities calculated by the Perkin & Walker formula.

Recovered on cruise IOSOP84-14.

--------------------------------------------------------------------------------

** VT3, VT4, VT5 and VT6 **

Tofino

VT3_0030.Cur -- 30m

RCM5 5101/5

Instrument depth is nominal. RCM is owned by U. Washington.

Channel 4 is uncalibrated.

Salinities calculated by the Perkin & Walker formula.

Recovered on cruise IOSOP84-14.

--removed Unknown channel

VT3_0050.CUR -- 50m

RCM4 4167/8- very bad translation, many bits can be seen to be  missing from

looking at the scope. Selector switch was found to be broken.

Only 3 good days of data. Assumed to be on time.

Instrument depth is nominal.

Salinities calculated by the Perkin & Walker formula.

Channel 4 is an uncalibrated channel.

RCM #4167 is owned by U. Washington.

Recovered on cruise IOSOP84-14.

--removed Unknown channel

VT3_0075.CUR -- 75m

RCM5 5098/8 low voltage on recovery.                         Suspicious fresh

water bulge #9370-#9435 padded out sal only. Many unusually high temperatures

Instrument depth is nominal.  Channel 4 is uncalibrated or unassigned.

Cond ch is "blocky" first two thirds of record.

Salinities calculated by the Perkin & Walker formula.

CM is owned by U. Washington.

Recovered on cruise IOSOP84-14.

--removed Unknown channel

VT4_0030.CUR -- 30m

RCM4 639/3 Instrument depth is nominal.  Temperature (Ch. 2) calibration is

approximate.  Channels 3 & 4 are uncalibrated (ch3 is cond? ch4 pr?).

Recovered on cruise IOSOP84-14.

--removed two Unknown channels

VT4_0050.CUR -- 50m

RCM5 5100/84 Battery low on recovery.

Coarse temperature calibration is approximate.

Instrument depth is nominal.  RCM is owned by U. Washington.

Channels 3 & 4 are unassigned; 0's and 1023's respectively.

Recovered on cruise IOSOP84-14.

--removed two No Sensor channels

VT4_0100.CUR -- 100m

RCM5 5901/2 CM is owned by U. Washington. Instrument depth is nominal.

Coarse temperature calibration is approximate.

Channels 3 & 4 are unassigned; 0's and 1023's respectively.

Recovered on cruise IOSOP84-14.

--removed two No Sensor channels

VT5_0030.CUR -- 30m

RCM4/L 3667/9  Temperature is mostly garbage.  The pressure  drifts steadily

up, making the occasional jump. Pressure excursions are typically 3 db.

Instrument depth is nominal.  Salinity is suspect; large (48.6) values were

recorded. Sp/dir histogram has only one peak at 136deg; nothing at 330.

Salinities calculated by the Perkin & Walker formula.

Recovered on cruise IOSOP84-14.

VT5_0050.CUR -- 50m

RCM4 1566/16  RCM is owned by U. Washington.                    Coarse temper

ature calibration is approximate

Instrument depth is nominal.

Channels 3 & 4 are unassigned; 0's and 1023's respectively.

Dominant flow direction is 130deg; no reverse component.

Recovered on cruise IOSOP84-14.

--removed two No Sensor channels

VT5_0100.CUR -- 100m

RCM4 525/11 6000:1 gear box. synch pulse appeared funny at end. The speed cha

nnel was edited before calibrating. added 20 min to start time to correct for

bug in ANDR6.   CM is owned by U. Washington.

Instrument depth is nominal.  Ch 3 is unassigned; Ch 4 appears to be an

uncalibrated pressure channel.

Recovered on cruise IOSO84-14..

--removed No Sensor and Unknown channels

VT5_0150.CUR -- 150m

AANDERAA 4400/1 CM is owned by IOS (OP/Fr. Seas).

Instrument depth is nominal.

1408 records were padded in ch 4 (Ex. Temp.)

Salinities calculated by the Perkin & Walker formula.

Bi-modal flow at 185/325 degrees.

Recovered on cruise IOSOP84-14.

VT6_0020.CUR -- 20m

RCM4 4316/3

Coarse temperature calibration is approximate.

Channel 3 is uncalibrated; ch 4 looks like uncalibrated pressure.

Instrument depth is nominal.

Bi-modal flow: 330/100 deg.

Recovered on cruise IOSOP84-14.

--removed two Unknown channels

--------------------------------------------------------------------------------

** W01 and W02 **

Mukkaw Bay (north)and West of Washington State Coast (south)

W01_0030.CUR -- 30m

RCM4 7460/1  Instrument hit July 28 - vane broken; thereafter directions are

padded.  Instrument depth is nominal.

Channel 3 & 4 were unknown/uncal; ch 4 probably Ex.Temp, over-ranged.

CM is owned by UBC.

Bi-modal flow: 185/40 deg.

Recovered on cruise IOSOP84-14.

--removed two Unknown channels

W01_0045.CUR -- 45m

RCM4 7461 Top instrument in mooring was hit on July 28; not apparent in this

record. Instrument depth is nominal.  Ch 3 is uncalibrated (Cond?); ch 4 is

over-ranged (Temp?).                  SY    9862LR       0SR       0AB

0BE       0PE       0

RCM is owned by UBC.

Recovered on cruise IOSOP84-14.

--removed two Unknown channels

W02_0030.CUR -- 30m

RCM5 5970/5 recovered Nov 28/84.  Chan 4 (exp temp) overranged  so padded out

.  Also padded out cond spikes in ch 3. One record was deleted to correct the

time.                                   11883:SY      0:LR      2:SR      0:A

B      1:BE      1:PE

Salinities calculated by the Perkin & Walker formula

Instrument depth is nominal.

Recovered on cruise IOSOP84-15.

--removed Raw Temp fine channel

W02_0050.CUR -- 50m

RCM5 5971/4  recovered Nov 28/84 . Many speed spikes and dropouts were interp

olated.  Conductivity  spikes were padded.         expanded temp (ch4) overra

nged, so padded out.                    11806:SY     74:LR     75:SR      2:A

B    108:BE     83:PE

Salinities calculated by the Perkin & Walker formula.

Instrument depth is nominal.

Recovered on cruise IOSOP84-15.

--removed Raw Temp fine channel

W02_0065.CUR -- 65m

RCM4 2520/1  recovered Nov 28/84. Coarse temp cal is appx.      Temp & direct

ion signal change after #7840.  Direction is very  suspicious, large shifts 8

529-8749, 9176-     , 11024-11257.      11658:SY      0:LR      0:SR      1:A

B      2:BE      1:PE

Instrument depth is nominal.

RCM is owned by U. Washington.

Recovered on cruise IOSOP84-15.

--removed Unknwon and No Sensor channels

--------------------------------------------------------------------------------