- TCWL or TCF version 2 Format

The TCWL or TCF version 2 (TCFv2) format files can contain any type of data which has 12 or fewer parameters. These files have a header of 24 lines with 80 characters in each line include the line feed character. The last two visible characters in each header line are ||. Because the definition of the header fields is very precise do not edit the header of these files with a plain text editor. Although the data attached to the header can be either short binary, long binary or ASCII, only the ASCII case will be discussed here. Several forms of ASCII data and their formats are used by the TCWL Tools, these are

- harmonic constituent files, waterlevel and current

- time series data file, waterlevel and current

- daily maximum and minimums (sometimes called extremes) data files, waterlevel and current

The follow is an example of the TCFv2 format file header and one line of data:

WaterLevel      08735 WINTER HARBOUR                               1999/05/01||

!Predicted   50 31.0   N 128 02.0   W                        +08   0000:00   ||

1 0001days 100.0% R                                       0100:00 03||

Stephenson,FE                                                                ||

Reference      052         00.0 05.0                                        0||

NAD 27 Chart                         ||

01 Date      YYYY/MM/DD     0       TY                                       ||

02 Time           hh:mm     0       TY                                       ||

03 Water Level        m     0       TY                                       ||

||

||

||

Tides & Currents, IOS     Shore                                              ||

08735p19990501a.wl60      08735.log                 fl__08735_yyyymmdda.wlev ||

Constituents are from Crawford's analysis at IOS 3 Nov 1997.                 ||

Z0 from Stephenson's analysis updated Feb 1996.                              ||

Checked against Crawford's results. OK BdLB 14 July 1998.                    ||

Doodson numbers as defined by Godin's The Analysis of Tides. pp 25-27        ||

5............................................................................||

6............................................................................||

7............................................................................||

8............................................................................||

9............................................................................||

10...........................................................................||

1999/05/01 00:00   3.500

- Detailed description of header format

Most users of this data will need only some of the information available from the header. The key header fields are

- The date, time and time zone of the data record start

- The location of the measurement site, in latitude and longitude

- The location of the measurement site, as a name

- The number of data records in a file

- The time interval (measurement interval) between data records

- The number of parameters or different kinds of information in each data record

- A description of the parameters and their order

- The depth below sea surface or chart datum the data was measured at

A detailed header description follows, mandatory and highly recommended entries are in bold:

''''/''''1''''/''''2''''/''''3''''/''''4''''/''''5''''/''''6''''/''''7''''/''''

DataType....... Index GeographicalName....................  DpthIm YYYY/MM/DD||

SOb/Pr/Com   DD MM.MMMMN DDD MM.MMMMW PstnkmA FixMethod..... +TZ.. hhmm:ss   ||

NumbRec... ####days ###.#% T TimeComments........................  hhmm:ss Ch||

Scientist............. ProjectID............. CruiseID..  IDAcA  WDAcA DpthWm||

FreeFieldForTemporaryStorage............................................. VCL||

hhmm:ss Cut.Offday Offs.etthbp1 Fld Ebb HorDat VertlDatum Bm_Name... BmElevmm||

01 Parameter Descriptor Accry Qulty #A 07 Parameter Descriptor Accry Qulty #A||

02 .................... ..... ..... .. 08 .................... ..... ..... ..||

03 .................... ..... ..... .. 09 .................... ..... ..... ..||

04 .................... ..... ..... .. 10 .................... ..... ..... ..||

05 .................... ..... ..... .. 11 .................... ..... ..... ..||

06 .................... ..... ..... .. 12 .................... ..... ..... ..||

Agency................... Platform................. InstrumentType....Serial#||

FileName................. ProcessingFileName....... ParentFile...............||

Comments.....................................................................||

.............................................................................||

.............................................................................||

.............................................................................||

.............................................................................||

.............................................................................||

.............................................................................||

.............................................................................||

.............................................................................||

.............................................................................||

''''/''''1''''/''''2''''/''''3''''/''''4''''/''''5''''/''''6''''/''''7''''/''''

The following is a full description of the format by line number. Each description consists of: the character position, field length and the definition. If a field position is not defined it is to be left blank.

Line 1:

col 01-15 15 : Type of data [See standard list], [File may contain other parameters]

17-21  5 : Station ID or Index number [no blanks]

23-58 36 : Geographical name [may include parenthetical remarks, i.e. (xxx)]

61-65  5 : Depth of instrument below chart datum [no blanks]

66-66  1 : Units of depth  [m: metres, f: feet]

68-71  4 : Date (Year) of first data record

72-72  1 : '/'

73-74  2 : Date (Month) of first data record

75-75  1 : '/'

76-77  2 : Date (Day) of first data record

Line 2:

col 01-01  1 : Archive status ! = CHS is primary archivist

M = MEDS is primary archivist

& = CHS is secondary archivist, primary archive elsewhere

? = Unknown

02-10  9 : 'Observed ', 'Predicted' or 'Computed '

14-15  2 : Latitude (Degrees)

17-23  7 : Latitude (Minutes up to 4 places of decimal)

24-24  1 : 'N' or 'S'

: 26-28  3 Longitude (Degrees)

30-36  7 : Longitude (Minutes up to 4 places of decimal)

37-37  1 : 'W' or 'E'

39-42  4 : Position accuracy

43-44  2 : Position accuracy units (m, km, etc.)

45-45  1 : Position quality flag

47-60 14 : Fix method, e.g. GPS, DGPS, Radar Range, etc.

62-66  5 : Time Zone [# of hours to add to determine UTC, always include + or - and

always left justify, (leaves space for Nfld. time). i.e. +03.5]

68-69  2 : Time (Hour)   of first data record

70-71  2 : Time (Minute) of first data record

72-72  1 : ':'

73-74  2 : Time (Second) of first data record

Line 3:

col 01-10 10 : Number of Records to follow header

12-15  4 : Elapsed time in days

16-19  4 : 'days'

21-25  5 : Percentage of primary data (DataType) not padded with nulls. Any parameter can

be padded to account for missing or poor data. The padded value depends upon the

file format, where:

Null = 9999999999 for ASCII data

Null = 32767 for 16 bit binary [(2**15)-1]

Null = 2147483647 for 32 bit binary [(2**31)-1]

26-26  1 : '%'

28-28  1 : Time Flag [See standard list]

30-65 36 : Time Comments [See standard list for suggestions] If time accuracy is needed

then the time accuracy is written at the end of the time comments.  the time accuracy

must be preceded by a sign (- or +) to be recognized. The units of time accuracy will

be seconds/day with at least one decimal place, i.e. +1.0 is 1 second/day.

68-69  2 : Sampling interval (Hours)

70-71  2 : Sampling interval (Minutes)

72:72  1 : ':'

73-74  2 : Sampling interval (Seconds)

76-77  2 : Number of Parameters in a record [up to 12]

Line 4:

col 01-22 22 : Name of scientist [See standard list]

24-45 22 : Project ID [See standard list]

47-56 10 : Cruise ID [See list of suggested entries]

59-62  4 : Instrument Depth accuracy number, real number with moving decimal always

in units of the depth

63-63  1 : Instrument Depth accuracy code

66-69  4 : Water Depth accuracy number, real number with moving decimal always in

units of the depth

70-70  1 : Water Depth accuracy code

72-76  5 : Depth of water below chart datum [no blanks]

77-77  1 : Units of depth [m: metres, f: feet]

Line 5:

col 01-73 73 : Free field for program scratch or

temporary storage (at present contains some reserved

fields -- details are given at end of this

section, section 4

75-77  3 : Number of additional comment lines.  Extra comment lines are still 77 characters

long with || at the end. Max. number of additional comment lines is 999.

Line 6:

Col 01-07  7 : Sampling interval of original measurements before subsampling or

interpolation.

09-15  7 : Cut-off for the filter, with decimals possible

16-18  3 : Units of the cut-off: yr, day, hr, min, sec

20-27  8 : Offset (this includes the usual pressure gauge Removed Mean) - format is

ddd.ddd.

28-29  2 : Offset units, usually db (dbars)

30-31  2 : Parameter offset is from.

33-35  3 : Flood direction(deg. T)

37-39  3 : Ebb direction(deg. T)

41-46  6 : Horizontal Datum, e.g. NAD-83

48-57 10 : Vertical Datum, e.g. GVSD91

59-68 10 : Bench Mark Name

70-75  6 : Bench Mark Elevation

76-77  2 : Units of BM Elevation [mm: millimetres, hf: ft*100]

Line 7 to Line 12:

col 01-02  2 : Parameter Number

40-41  2 :     "       "

04-23 20 : Parameter Descriptors [See standard list]

43-62 20 :     "          "        "    "

25-29  5 : Parameter Accuracy  [uncertainty, using same number

64-68  5 :    "         "      of decimal places as in c37 or c76]

31-35  5 : Flags to describe parameter quality, degree of editing, and if applicable type of

datum and filters applied.

70-74  5 : Same flags as c31-c35

37-37  1 : Number of decimal places or 'T' if text (see note re C and W data format)

76-76  1 :    "    "    "      "    "   "  "   "

38-38  1 : Data Format      where 	A = ASCII  [10 bytes/parameter fixed format]

77-77  1 :   "    " 		T = Tide Table event file format

C = Current Constituent file format (ASCII)

W = Water Level Constituent format (ASCII)

Note: for details of C and W data formats see the Constituent File Data Format

Specification section that follows below.

[the following are for internal use, that is, not for interchange:

S = (16 bits Integer Binary)

L = (32 bits Integer Binary)

V = (16 bits VAX Int Binary)

X = (32 bits VAX Int Binary)]

Line 13:

01-25 25 : Data set Agency [See standard list]

27-51 25 : Platform used for measurements [See standard list]

53-70 18 : Instrument Type [See standard list]

71-77  7 : Instrument Serial Number

Line 14:

01-25 25 : File Name [Name of present file with extension]

27-51 25 : Processing File Name [Log file, could be paper]

53-77 25 : Parent File Name [Source file name with extension]

Line 15 to Line 24:

01-77 77 : Comments -- there are always at least 10 lines of comments even if left blank.

Line 25 to Line ???

01-77 77 : Variable length Comments - each line must be a full line with a maximum of

999 lines (not implemented in TCF routines (October 20, 1994).

- Note:

- All header lines are terminated by '||' and an End-of-Line character(s). This means each header line has a length of 79 characters plus EOL character(s).

- If data format is standard ASCII the record length will be 10 bytes * number of parameters plus end of line. A special ASCII format is used for harmonic constituent files.

- If data format is binary the record length will be 2 or 4 bytes * number of parameters and no end of line.

- Scratch Field:

The scratch field is used in various programs. Specific use is as follows:

''''/''''1''''/''''2''''/''''3''''/''''4''''/''''5''''/''''6''''/''''7''''/''''

PrdctType vx.x pgn SpFn    mi.n ma.x                         dh tmx RefID VCL||

Explanation:

vx.x

version number of the data. Where each x is a number. (Used to specify the constituent set version number. Also used in T&C Table Production for event file version)

SpFn

speed type of station (i.e. FAST or NORM ) or Foot note number (i.e. FTN1 which is the foot note for Grondines)

pgn

page number of the first page that the predictions will appear on in the tide table

PrdctType

type of station -- either Reference, Secondary, or Other

tmx

max time span of a series of 3 small changes in minutes

RefID

Index number of the Secondary Port’s reference port

mi.n

minimum waterlevel value (in metres) for the calendar plots

ma.x

maximum waterlevel value (in metres) for the calendar plots. The max - min must be divisible with one decimal place by 5, ie 6 - 0 works, 6 - 0.25 will not work. )See below for details).

dh

Water levels xx amplitiude of a small change in cm (ie less than 99 cm) in automatic editing
Currents (blank) do not use central time in calculation in automatic editing
Currents CT use central time calculation in automatic editing and make it marked for attention
Currents CF use central time calculation in automatic editing and make it active

Maximum and minimum waterlevel values presently used in the Tide and Current Tables:

| Volume | Minimum | Maximum |
| --- | --- | --- |
| 5 | 0.0 | 4.0 |
| 6 | 0.0 | 6.0 |

- 6. List of Standard Entries The lists that follow contain the standard entries as of 28 May 1996. Additional entries will be added as required. These entries have standardized spelling so that the corresponding header fields can be searched and various software can key on the entries for suitable responses. Thus it is important that only the standardized spellings be used. Note that in the headers the underscore character ‘_’ is actually a blank and should appear as such. The form used below with underscore is required for use in various C programs that read these in as strings from files containing the lists of standard entries. Data Types WaterLevel WaterLevExtrema WaterLevConstit WaterLevResid WaterLevREFDS CurrentMeter CurrentObserved CurrentExtrema CurrentConstit CurrentDPModel PressureWater PressureTotal PressureAtmos PressureConstit PressureResid DriftingBuoy Profile Wind Temperature None_oftheabove Instrument Types Aanderaa_RCM4 Aanderaa_RCM4S Aanderaa_RCM5 Aanderaa_RCM7 CMDR_(AML_mod) CMDR_(paper_tape) AML_RCM12 EG&G_VMCM InterOcean_S4 Marsh-McBirney_585 Endeco_174_CM Geodyne850 Geodyne850_AML_mod Neil_Brown_ACM-2 Dobrocky_Drifter Sea_Rover_2 Sea_Rover_3 Aanderaa_TG1A Aanderaa_TG2A Aanderaa_TG3A Aanderaa_WLR5 AML_TG12A AML_TG12B AML_750A Float_(OTT) Float_(TATS) Float_(MtrBurst) Float_(other) Ottboro Sutron(Bubbler) Sutron(Other) Bubbler_(Stev/Man) Bubbler_(River) Bubbler_(LPTG) Aanderaa_TR1 Aanderaa_TR2 AML_J-Tec Other(see_comment) None_of_the_above Data Owner Tides_&_Currents,_IOS Ocean_Physics,____IOS Ocean_Ecology,____IOS Ocean_Chemistry,__IOS Climate_Chemistry,IOS Tides_&_Water Levels,CCIW Tides_&_Water Levels,_BIO Marees_&_Courants,_IML MEDS-SDMM,_Ottawa Unknown None_of_the_above Data Collection Platform Mooring,_surface Mooring,_sub-surface Mooring,_type_unknown Ship,_PARIZEAU Ship,_TULLY Launch,_DEMON Surface_drifter Anchor,_mooring Anchor,_bottom Anchor,_bedrock Shore None_of_the_above Data Parameter or Variable Description Salinity________0/00 Salinity_sal-10_____ Salinity_35-sal_____ Reference___________ Temp_coars(t68)deg_C Temp_fine_(t68)deg_C Temperatur(t68)deg_C Temp_coars(t90)deg_C Temp_fine_(t90)deg_C Temperatur(t90)deg_C Sensortemp(t68)deg_C Sensortemp(t90)deg_C Cond_35-con____mS/cm Conductivity___mS/cm Conductivity_ratio__ Pressure_______dbars Direction______deg_T Speed____________m/s Speed__________knots Press_w/Atmos__dbars Water_Level________m Water_Level_______ft Time/Pressure__dbars E_component______m/s N_component______m/s Component________m/s Transmission_______% Accuracy____________ Padded______________ Temperature______raw Conductivity_____raw Temp_fine________raw Temp_coarse______raw Pressure_________raw Time/Pressure____raw Direction________raw Speed____________raw E_component______raw N_component______raw Transmission_____raw Overflw_bit_pressure Most_sig.___pressure Least_sig.__pressure Time_code___________ Hour________________ Minute______________ Second______________ HourMinute______hhmm Month_______________ Day_________________ MonthDay________MMDD Year____________YYYY Day_of_Year______DDD Buoy_id_#___________ Time_from_1/1/1970_s Time_from_start____s W_Loran-C_5990____us X_Loran-C_5990____us Y_Loran-C_5990____us Z_Loran-C_5990____us Longitude________deg Latitude_________deg Const_Name_[Ref_Nam] Nominal_Period_hours Doodson_Numbers Ellipse_Maj_Axis_m/s Ellipse_Min_Axis_m/s Ellipse_Inc______deg Amplitude_____metres Amplitude______dbars Phase_Lag[g]_____deg Phase_Lag[g+]____deg Phase_Lag[g-]____deg U_Cos_Fit___________ U_Sin_Fit___________ V_Cos_Fit___________ V_Sin_Fit___________ Begin_slack_________ End_slack___________ Direction_turn______ Security Unknown_____________ No_Sensor___________ None_of_the_above___ Project Names PERD,_Charlottes CODE_II CODE SuperCODE NCODE La_Perouse CCEXP Hot_Vents GEORGIA_STR._L.S.C. MASS Current_Surveys Wave-Currnt_Studies Narrow_Passes Alberni Pr._Rupert_Harbour Testing_Currents Testing_Tides Calibra_Currents Tidal_Net_Perm Tidal_Net_Temp Tsunami_Net Tidal_Hydrog/Temp Tidal_Submerged Unknown None_of_the_above Project Scientist Ages,AB Crawford,WR de_Lange_Boom,BR Huggett,WS Stephenson,FE Woodward,MJ Sandilands,R Solvason,R O'Reilly,C Labrecque,B Bolduc,P-A Thomson,RE Freeland,H Stucchi,D Mitchell,D Unknown NoneOftheAbove Written by: Bodo de Lange Boom, CHS Pacific Date: 17 Nov 1999 Converted to HTML: Anne Ballantyne, CHS Pacific Date: 28 March 2000 For more detailed information on the format of the 24 line or TCF version 2 header contact Anne Ballantyne (ballantynea@pac.dfo-mpo.gc.ca) or Bodo de Lange Boom (delangeboomb@pac.dfo-mpo.gc.ca) Filename format as provided by Anne Ballantyne February 27, 2014. Example file: A001_0002c70140a.CUR A001 is the mooring identifier. 0002 is the depth C70 means 1970 (c is 19) 140 is the day of the year (I believe in 1970 day 140 was May 20th) A means the first file with the rest of the name