Current meter/thermistors chain  processing notes

Jan 31, 1995

Mission :

Date    : Jul 15 - Jul 25, 94

Project : Hot vent 94 (Endeavor Ridge)

Station : ERA1 and ERA2

2 current meter data files processed.

1.  Calibration Coefficients

The calibration  coefficients are stored on PIMS.

2.  Editing

All of the data files needed editing.

3.  Particulars

CURRENT METERS:

STATION    STARTING          RECORDS  DEPTH(M)  INTERVAL   POS          CHANNELS

TAPE ID    ENDING TIME(UTC)

-------------------------------------------------------------------------------

ERA1       2220 JUL 15, 94   1402     2100      10 MINS    47 57.0  N   STCXS

5970/16    1550 JUL 25, 94                                129 05.2  W

Notes :  OK

DATA  :  CCS$PHYSICS7:[WORK.AYPLEE.HV]

Backup:  NO

--------------------------------------------------------------------------------

ERA2       2210 JUL 16, 94   1249     2110      10 MINS    47 56.7  N   STCPV

V900X5     1410 JUL 25, 94                                129 05.4  W

Notes :  OK

Andrew Lee

Data archived on tapes U02701 and U02711

--------------------------------------------------------------------------------

Current meter/thermistors chain  processing notes

May  8, 1995

Mission : Deployed : 9426

Revovered: By NOAA ship

Date    : July 26, 1994 - Aug 8, 1994

Project : Hot Vent 94 (Endeavour Ridge)

Station : CB02

1 RCM5 current meter data file processed.

1.  Calibration

COARSE Temperature Calibration: Calib Date  NOV 06,92      Channel=   2

CT0=-0.2494000E+01 CT1= 0.2243800E-01 CT2=-0.6341000E-06 CT3= 0.1545100E-08

Conductivity Calibration:       Calib Date  NOV 06,92      Channel=   3

C0= 0.2792300E+00  C1= 0.2028000E-02  C2=-0.2859000E-05  C3= 0.1623200E-08

FINE Temperature Calibration:   Calib Date  NOV 06,92      Channel=   4

XC0=-0.4466000E+00 XC1= 0.7752000E-02 XC2= 0.0000000E+00 XC3= 0.0000000E+00

Compass Corrections:            Calib Date  JUL 19,91      Channel=   5

20.0 19.0 19.0 18.0 18.0 18.0 18.0 18.0 17.0 17.0 17.0 17.0

17.0 17.0 18.0 17.0 17.0 17.0 18.0 18.0 18.0 19.0 19.0 20.0

21.0 21.0 21.0 21.0 21.0 20.0 21.0 21.0 20.0 20.0 19.0 19.0

Speed Calibrations:             Calib Date  CB02,HV94      Channel=   6

C0=  0.011000     C1= 0.007817     Gear Ratio= .50

2.  Editing

Minor editings.

3.  Particulars

CURRENT METERS:

STATION    STARTING          RECORDS  DEPTH(M)  INTERVAL   POS          CHANNELS

TAPE ID    ENDING TIME(UTC)

-------------------------------------------------------------------------------

CB02       2240 JUL 26, 94   1876     2633      10 MINS    47 45.0  N   STrXV

8729/5     2310 AUG 08, 94                                127 44.9  W

Notes :  OK

Andrew Lee

Data archived on tapes U02701 and U02711

Current meter  processing notes

Sept 19h, 1995

Mission : 9405 (deployment), U. of Washington's Attlantis II (recovery)

Date    : July 16, 1994 - May 14, 1995

Project : Hot Vent 94 (Endeavour Ridge, NW Pacific)

Station : CA01, CB01 and ERA2

Three current meter data files were processed.

1.  Raw files

One current meter data file each for CA01, CB01 and ERA2.

2.  Clip data

STATION    STARTING          RECORDS  DEPTH(M)  INTRVAL   POS        CHANS

TAPE ID    ENDING TIME(UTC)

---------------------------------------------------------------------------

Current meter:

CA01       0000 JUL 21, 94   7142     1840      60        47 56.4 N  RTrPV

8558/5     1300 MAY 14, 95                               129 09.7 W

OK

CB01       2200 JUL 16, 94   4745     1786      60        48 07.5 N  RT_PV

5382/4     1400 JAN 30, 95                               128 21.1 W

Encoder problem(?). Meter stopped working after 1400 Jan, 95

No conductivity sensor in this meter.

ERA2       0100 JUL 26, 94   7026     1786      60        47 57.0 N  RTrPV

V900/6     1800 MAY 14, 95                               129 05.7 W

OK

2.  Editing

Used program VIEW to edit.

3.  Calibration

processing date : 15-SEP-95 13:32:19

Input File Name : CA01_1840_940721_A.VIEW_CLIP

Output File Name : CA01_1840_940721_A.cvrt

Site: CA01     Depth = 1840 M Endeavour Ridge        Lat 4756.4N    Long 129 9.7W   Meter#   8558

Start Time is    0: 0: 0   21/ 7/94 UTC                                         Records =    7142

COARSE Temperature Calibration: Calib Date  MAR 10,93      Channel=   2

CT0=-0.2511000E+01 CT1= 0.2302400E-01 CT2=-0.2081000E-05 CT3= 0.2444400E-08

Conductivity Calibration:       Calib Date  MAR 10,93      Channel=   3

C0= 0.5370700E+00  C1= 0.4691600E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Pressure Calibrations:          Calib Date JUNE 16,94      Channel=   4

PC0=-0.1229000E+03 PC1= 0.2254300E+01 PC2= 0.0000000E+00 PC3= 0.0000000E+00

Compass Corrections:            Calib Date  MAR 25,91      Channel=   5

21.0 21.0 21.0 21.0 21.0 22.0 22.0 22.0 23.0 23.0 23.0 23.0

24.0 24.0 24.0 24.0 24.0 24.0 24.0 24.0 24.0 23.0 23.0 23.0

23.0 23.0 22.0 22.0 21.0 21.0 21.0 20.0 20.0 20.0 19.0 19.0

Speed Calibrations:             Calib Date CA01,HV-94      Channel=   6

C0=  0.011000     C1= 0.007817     Gear Ratio= 2.0

processing date : 15-SEP-95 16:27:35

Input File Name : CB01_1786_940716_A.CLIP

Output File Name : CB01_1786_940716_A.cvrt

Site: CB01     Depth = 1786 M Endeavour Ridge        Lat 48 7.5N    Long 12821.1W   Meter#   5382

Start Time is   22: 0: 0   16/ 7/94 UTC                                         Records =    4745

COARSE Temperature Calibration: Calib Date  MAY 14,92      Channel=   2

CT0=-0.2440000E+01 CT1= 0.2296400E-01 CT2=-0.2036000E-05 CT3= 0.2370600E-08

Conductivity Calibration:       Calib Date  MAY 14,92      Channel=   3

C0= 0.5447000E+00  C1= 0.4723300E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Pressure Calibrations:          Calib Date  JUN 16,94      Channel=   4

PC0=-0.1124000E+03 PC1= 0.2253700E+01 PC2= 0.0000000E+00 PC3= 0.0000000E+00

Compass Corrections:            Calib Date CB01,HV-94      Channel=   5

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

Speed Calibrations:             Calib Date CB01,HV-94      Channel=   6

C0=  0.015000     C1= 0.007000     Gear Ratio= 2.0

processing date : 18-SEP-95 10:03:21

Input File Name : ERA2_1757_940726_A.VIEW_CLIP

Output File Name : ERA2_1757_940726_A.cvrt

Site: ERA2     Depth = 1757 M Endeavour Ridge        Lat 4757.0N    Long 129 5.7W   Meter#   V900

Start Time is    1: 0: 0   26/ 7/94 UTC                                         Records =    7026

COARSE Temperature Calibration: Calib Date  DEC 13,90      Channel=   2

CT0=-0.2655000E+01 CT1= 0.2296600E-01 CT2=-0.1565000E-05 CT3= 0.2008100E-08

Conductivity Calibration:       Calib Date  DEC 13,90      Channel=   3

C0= 0.5359400E+00  C1= 0.4733100E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Pressure Calibrations:          Calib Date  JUN 16,94      Channel=   4

PC0=-0.1130000E+03 PC1= 0.2250400E+01 PC2= 0.0000000E+00 PC3= 0.0000000E+00

Compass Corrections:            Calib Date  MAR 19,91      Channel=   5

20.0 21.0 20.0 20.0 20.0 20.0 20.0 19.0 19.0 18.0 18.0 18.0

19.0 19.0 20.0 20.0 21.0 22.0 23.0 23.0 24.0 23.0 23.0 23.0

23.0 23.0 23.0 22.0 21.0 20.0 19.0 19.0 19.0 19.0 18.0 18.0

Speed Calibrations:             Calib Date  ERA2,HV94      Channel=   6

C0=  0.011000     C1= 0.007817     Gear Ratio= 2.0

4. Editing after calibration

Padded channel 3 on

5. Direction average

Direction averaged all current meter files. file names are *.CUR

6. Harmonic Analysis

The results are printed out and stored on files with extensions *.HAR_LOG

7. Standard monthly plot

The standard monthly plots are done on all the files.

--------------------------------------------------------------------------------

Andrew Lee