Current meter  processing notes

Sept 11th, 1995

Mission : 9431 (deployment)

9501 (deployment) for the second mooring for G station

9507 (recovery)

Date    : Aug 13, 1994 - April 29, 1995

Project : JGOFS

Station : G (2 moorings) O and S.

Four current meter data files file were processed.

1.  Raw files

Two current meter data files for G (2 moorings).

One current meter data file each for O and S.

2.  Clip data

STATION    STARTING          RECORDS  DEPTH(M)  INTRVAL   POS        CHANS

TAPE ID    ENDING TIME(UTC)

---------------------------------------------------------------------------

Current meter:

G          0330 AUG 13, 94   3198     50        30        48 38.8 N  RTr%V

1014/G-T   1800 OCT 18, 94                               127 38.3 W

Mooring was returned by fisherman. Recovered drifting Oct 18, 1994.

G          2030 FEB 21, 95   3172     50                  48 38.8 N  RTr%V

1014/2     2200 APR 28, 95                               127 38.3 W

This was the second mooring for G station.

O          0330 AUG 14, 94   12419    50                  49 07.5 N  RTr%V

6754/3     2030 APR 29, 95                               127 44.1 W

Transmissometer stopped working after Nov 30, 1994.

S          0130 AUG 13, 94   7471     50                  48 47.1 N  RTr%V

7337/20    1630 JAN 17, 95                               127 58.6 W

Transmissometer stopped working after Dec 3, 1994.

Dead battery caused meter to stopped early.

2.  Editing

Used program VIEW to edit.

3.  Calibration

processing date :  8-SEP-95 14:51:08

Input File Name : G____0050_940813_A.VIEW_CLIP

Output File Name : G____0050_940813_B.cvrt

Site: G        Depth =   50 M Kyuquot Sound (near shore)Lat 4938.8N  Long 12738.3W   Meter#   1014

Start Time is    3:30: 0   13/ 8/94 UTC                                          Records =    3198

COARSE Temperature Calibration: Calib Date  SEP 29,93      Channel=   2

CT0=-0.2468000E+01 CT1= 0.2278800E-01 CT2=-0.1639000E-05 CT3= 0.2174200E-08

Conductivity Calibration:       Calib Date  SEP 29,93      Channel=   3

C0= 0.5596800E+00  C1= 0.4910300E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Transmissivity Calibrations :   Calib Date  G,JGOFS94      Channel=   4

XV0=-0.6442000E-01 XV1= 0.4842200E-02 XV2= 0.0000000E+00 XV3= 0.0000000E+00

XVA= 0.4610000E+01 XVB= 0.4610000E+01 XVZ= 0.0000000E+00

Compass Corrections:            Calib Date  G,JGOFS94      Channel=   5

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

Speed Calibrations:             Calib Date  G,JGOFS94      Channel=   6

C0=  0.015000     C1= 0.007000     Gear Ratio= 4.0

processing date :  1-SEP-95 14:27:11

Input File Name : G____0050_950221_A.VIEW_CLIP

Output File Name : G____0050_950221_A.cvrt

Site: G        Depth =   50 M Kyuquot Sound (near shore)Lat 4938.8N  Long 12738.3W   Meter#   1014

Start Time is   20:30: 0   21/ 2/95 UTC                                          Records =    3172

COARSE Temperature Calibration: Calib Date  SEP 29,93      Channel=   2

CT0=-0.2468000E+01 CT1= 0.2278800E-01 CT2=-0.1639000E-05 CT3= 0.2174200E-08

Conductivity Calibration:       Calib Date  SEP 29,93      Channel=   3

C0= 0.5596800E+00  C1= 0.4910300E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Transmissivity Calibrations :   Calib Date  G,JGOFS94      Channel=   4

XV0=-0.3289000E+00 XV1= 0.5150000E-02 XV2= 0.0000000E+00 XV3= 0.0000000E+00

XVA= 0.4610000E+01 XVB= 0.4610000E+01 XVZ= 0.0000000E+00

Compass Corrections:            Calib Date  G,JGOFS94      Channel=   5

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

Speed Calibrations:             Calib Date  G,JGOFS94      Channel=   6

C0=  0.015000     C1= 0.007000     Gear Ratio= 4.0

processing date :  5-SEP-95 11:24:56

Input File Name : O____0050_940814_B.VIEW_CLIP

Output File Name : O____0050_940814_A.cvrt

Site: O        Depth =   50 M Nootka Sound (offshore)   Lat 49 7.5N  Long 12744.1W   Meter#   6754

Start Time is    3:30: 0   14/ 8/94 UTC                                          Records =   12419

COARSE Temperature Calibration: Calib Date  SEP 29,93      Channel=   2

CT0=-0.2468000E+01 CT1= 0.2278800E-01 CT2=-0.1639000E-05 CT3= 0.2174200E-08

Conductivity Calibration:       Calib Date  SEP 29,93      Channel=   3

C0= 0.5526500E+00  C1= 0.4867400E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Transmissivity Calibrations :   Calib Date  JUN 29,93      Channel=   4

XV0= 0.3753600E+01 XV1= 0.1220000E-02 XV2= 0.0000000E+00 XV3= 0.0000000E+00

XVA= 0.4674000E+01 XVB= 0.4670000E+01 XVZ= 0.0000000E+00

Compass Corrections:            Calib Date  JUN 29,93      Channel=   5

23.0 22.0 22.0 22.0 22.0 22.0 23.0 23.0 23.0 24.0 24.0 24.0

24.0 24.0 24.0 24.0 24.0 24.0 24.0 24.0 24.0 23.0 23.0 23.0

23.0 23.0 23.0 23.0 23.0 22.0 22.0 22.0 22.0 23.0 23.0 23.0

Speed Calibrations:             Calib Date  O,JGOFS94      Channel=   6

C0=  0.015000     C1= 0.007000     Gear Ratio= 4.0

processing date :  6-SEP-95 15:32:55

Input File Name : S____0050_940815_A.VIEW_CLIP

Output File Name : S____0050_940815_B.cvrt

Site: S        Depth =   50 M Estevan Point (Offshore)  Lat 4847.1N  Long 12758.6W   Meter#   7337

Start Time is    1:30: 0   15/ 8/94 UTC                                          Records =    7471

COARSE Temperature Calibration: Calib Date  MAR 22,91      Channel=   2

CT0=-0.2104000E+01 CT1= 0.2266900E-01 CT2=-0.1071000E-05 CT3= 0.1714600E-08

Conductivity Calibration:       Calib Date  MAR 22,91      Channel=   3

C0= 0.5695400E+00  C1= 0.4568100E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Transmissivity Calibrations :   Calib Date S,JGOFS-94      Channel=   4

XV0= 0.0000000E+00 XV1= 0.4883700E-02 XV2= 0.0000000E+00 XV3= 0.0000000E+00

XVA= 0.4749000E+01 XVB= 0.4410000E+01 XVZ= 0.2000000E-02

Compass Corrections:            Calib Date S,JGOFS-94      Channel=   5

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

Speed Calibrations:             Calib Date S,JGOFS,94      Channel=   6

C0=  0.015000     C1= 0.007000     Gear Ratio= 4.0

4. Editing after calibration

Used VIEW to edit the salinity files. (*.SAL -> *.VIEW_SAL)

Starting 0500 Feb 22, 1995, directions from CRD1_0055_940819_A.SAL

were padded to -99999.000. Output file is CRD1_0055_940819_A.VIEW_SAL.

5. Direction average

Direction averaged all current meter files. file names are *.CUR

6. Harmonic Analysis

The results are printed out and stored on files with extensions *.HAR_LOG

7. Standard monthly plot

The standard monthly plots are done on all the files.

--------------------------------------------------------------------------------

Andrew Lee