Current meter processing notes

Oct 10th, 1996

Mission : 9312 (deployment), 9335 (recovery)

Date    : May 03, 1993 - Sep 05, 1993 pst.  First part of data.

Sep 05, 1993 - Jan 18, 1994 pst.  Second part of data.

Project : LAP-17

Station : E01

Comments:

Six current meter data files(part a and b) were processed.

The mooring was hit by the barge after 1600 pst Sep 5, 1993 and dragged to

a deeper(+20m) location.

The mooring was released (no date or time recorded) but did not surface.

The mooring was found and returned to IOS by a fisherman in July 8, 1996.

E01:

8833x6-a at  25m. Data are in good quanlity.

7996x6-a     35m.          "

8830x5-a     70m,          "

8833x6-b     45m. Rotor damaged, no current speed.

7996x6-b     55m.          "

8830x5-b     90m. Data are in good quanlity.

1.  Convert raw data files

Using IOSSHELL to add headers to raw data files.

2.  Clip data

STATION    STARTING          RECORDS  DEPTH(M)  INTRVAL   POS        CHANS

TAPE ID    ENDING TIME(PST)

---------------------------------------------------------------------------

Current meter:

E01        1600 MAY 03, 93   6001     25        30        49 17.7 N  RTrPV

8833/6-a   1600 SEP 05, 93                               126 37.3 W

E01        1600 MAY 03, 93   6001     35        30        49 17.7 N  RTrXV

7996/6-a   1600 SEP 05, 93                               126 37.3 W

E01        1600 MAY 03, 93   6001     70        30        49 17.7 N  RTrXV

8830/5-a   1600 SEP 05, 93                               126 37.3 W

--------           ---------          ---------

E01        1900 SEP 05, 93   5373     45        30        49 18.0 N  RTrPV

8833/6-b   1700 DEC 26, 93                               126 44.2 W

E01        1900 SEP 05, 93   56477    55        30        49 18.0 N  RTrXV

7996/6-b   1700 JAN 18, 94                               126 44.2 W

E01        1900 SEP 05, 93   6579     90        30        49 18.0 N  RTrXV

8830/5-b   2000 JAN 20, 94                               126 44.2 W

3.  Editing

Used program VIEW to edit.

4.  Calibration

Used program CALIB to calibrate data.  Coefficients are stored on CCF files

and also in the headers.

5. Editing after calibration

Removed current speed channel from 8833x6-b and 7996x6-b.

6. Moving average and hourly data

Moving average by PROG$MOVAVG3P. (*.CUR > *.F3P)

Thinned to hourly values. (*.F3P > *.THN)

7. Harmonic Analysis

In 9-line header format.

8. Standard monthly plot

Plotted.

--------------------------------------------------------------------------------

Andrew Lee

Data archived on tapes DLT017 and DLT018 in saveset lap17_b.bak

----------------------------------------------------------

Nov 12/96 RMb - new version of BP1_0035.CUR with recalculated CR/Salinity values

added to archive (replaces  old version.  Sea Andrew Lee for details.

---------------------------------------------------------------------------------