Current meter/Thermistors chain  processing notes

Aug 22nd, 1995

Mission : 9431 (deployment), 9507 (recovery)

Date    : Aug 18, 1994 - May 2, 1995

Project : La  Perouse 20

Station : A1 and E01

Seven current meter data files and 1 thermistors chain data file were processed.

1.  Raw files

Four current meter data files and one thermistors chain data file from A1.

Three current meter data files from E01.

2.  Clip data

STATION    STARTING          RECORDS  DEPTH(M)  INTRVAL   POS        CHANS

TAPE ID    ENDING TIME(UTC)

---------------------------------------------------------------------------

Current meter:

A1         1830 AUG 18, 94   12331    35        30        48 32.1 N  RTrPV

6101/24    1530 MAY 02, 95                               126 12.5 W

Channel 4 contains time code/pressure signals. Pressure values were

interpolated.

A1         1830 AUG 18, 94   7542     100       30        48 32.1 N  RTrPV

333/28     2100 JAN 22, 95                               126 12.5 W

Dead battery caused meter to stop early.

A1         1830 AUG 18, 94   10714    175       30        48 32.1 N  RTrXV

3502/12    2300 MAR 23, 95                               126 12.5 W

Dead battery caused meter to stop early.

Flipped directions 180 degrees.

A1         1830 AUG 18, 94   7082     400       30        48 32.1 N  RTrPV

7732/15    0700 JAN 13, 95                               126 12.5 W

Dead battery caused meter to stop early.

E01        0430 AUG 16, 94   302      25        30        49 17.0 N  RTrPV

3506/      1200 AUG 22, 94                               126 35.6 W

Tape jammed, stopped early.

E01        0430 AUG 16, 94   12228    35        30        49 17.0 N  RTrPV

5361/5     2230 APR 27, 95                               126 35.6 W

Ran out of tape.

E01        0430 AUG 16, 94   12406    75        30        49 17.0 N  RTrXV

8211/14    1500 MAY 01, 95                               126 35.6 W

OK

Thermistors chain:

A1         1900 AUG 18, 94   4722     102,107,..60        48 32.1 N  RTTTTT

889/7      1200 MAR 12, 95            152                126 12.5 W  TTTTTT

Dead battery caused meter to stop early.

2.  Editing

Used program VIEW to edit.

3.  Calibration

processing date : 14-JUL-95 07:21:08

Input File Name : A1___0035_940818_B.VIEW_CLIP

Output File Name : A1___0035_940818_A.cvrt

Site: A1       Depth =   35 M La Perouse Bank        Lat 4832.1N    Long 12612.5W   Meter#   6101

Start Time is   18:30: 0   18/ 8/94 UTC                                         Records =   12331

COARSE Temperature Calibration: Calib Date  AUG 20,91      Channel=   2

CT0=-0.2475000E+01 CT1= 0.2289800E-01 CT2=-0.1639000E-05 CT3= 0.2052300E-08

Conductivity Calibration:       Calib Date  AUG 20,91      Channel=   3

C0= 0.5677300E+00  C1= 0.4694800E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Pressure Calibrations:          Calib Date  AUG 23,91      Channel=   4

PC0=-0.9719000E+01 PC1= 0.1489800E+00 PC2= 0.0000000E+00 PC3= 0.0000000E+00

Compass Corrections:            Calib Date  AUG 27,91      Channel=   5

20.0 21.0 21.0 21.0 22.0 22.0 22.0 22.0 21.0 20.0 20.0 20.0

20.0 20.0 20.0 20.0 19.0 19.0 19.0 19.0 19.0 19.0 19.0 19.0

19.0 19.0 19.0 19.0 20.0 20.0 19.0 20.0 19.0 19.0 18.0 19.0

Speed Calibrations:             Calib Date  A1,LAP-19      Channel=   6

C0=  0.011000     C1= 0.007700     Gear Ratio= 4.0

processing date : 14-JUL-95 09:55:32

Input File Name : A1___0100_940818_A.VIEW_CLIP

Output File Name : A1___0100_940818_A.cvrt

Site: A1       Depth =  100 M La Perouse Bank        Lat 4832.1N    Long 12612.5W   Meter#    333

Start Time is   18:30: 0   18/ 8/94 UTC                                                Records =    7542

COARSE Temperature Calibration: Calib Date DEC 04,87       Channel=   2

CT0=-0.2471000E+01 CT1= 0.2258600E-01 CT2=-0.1407000E-05 CT3= 0.2110700E-08

Conductivity Calibration:       Calib Date DEC 04,87       Channel=   3

C0= 0.5522100E+00  C1= 0.4926900E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Pressure Calibrations:          Calib Date AUG 23,91       Channel=   4

PC0=-0.1900000E+02 PC1= 0.3766000E+00 PC2= 0.0000000E+00 PC3= 0.0000000E+00

Compass Corrections:            Calib Date JUL 30,91       Channel=   5

24.0 24.0 24.0 25.0 25.0 25.0 25.0 25.0 25.0 25.0 25.0 25.0

25.0 26.0 25.0 25.0 25.0 24.0 24.0 23.0 23.0 23.0 22.0 22.0

22.0 22.0 21.0 21.0 21.0 21.0 20.0 20.0 21.0 21.0 21.0 21.0

Speed Calibrations:             Calib Date  A1, LAP20      Channel=   6

C0=  0.011000     C1= 0.007817     Gear Ratio= 4.0

processing date : 31-JUL-95 08:32:51

Input File Name : A1___0175_940818_A.VIEW_CLIP

Output File Name : A1___0175_940818_A.cvrt

Site: A1       Depth =  175 M La Perouse Bank        Lat 4832.1N    Long 12612.5W   Meter#   3502

Start Time is   18:30: 0   18/ 8/94 UTC                                         Records =   10714

COARSE Temperature Calibration: Calib Date AUG 20, 91      Channel=   2

CT0=-0.2536000E+01 CT1= 0.2286900E-01 CT2=-0.1536000E-05 CT3= 0.2056200E-08

Conductivity Calibration:       Calib Date AUG 20, 91      Channel=   3

C0= 0.5377600E+00  C1= 0.4676900E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

FINE Temperature Calibration:   Calib Date AUG 20, 91      Channel=   4

XC0= 0.3912400E+01 XC1= 0.7912100E-02 XC2= 0.0000000E+00 XC3= 0.0000000E+00

Compass Corrections:            Calib Date AUG 27, 91      Channel=   5

23.0 23.0 23.0 23.0 23.0 24.0 24.0 24.0 24.0 23.0 24.0 24.0

24.0 25.0 25.0 26.0 25.0 25.0 25.0 24.0 24.0 24.0 25.0 24.0

24.0 24.0 23.0 22.0 22.0 22.0 21.0 21.0 20.0 20.0 19.0 20.0

Speed Calibrations:             Calib Date  A1,LAP-20      Channel=   6

C0=  0.015000     C1= 0.007000     Gear Ratio= 4.0

processing date : 31-JUL-95 08:59:02

Input File Name : A1___0400_940818_A.VIEW_CLIP

Output File Name : A1___0400_940818_A.cvrt

Site: A1       Depth =  400 M La Perouse Bank        Lat 4832.1N    Long 12612.5W   Meter#   7732

Start Time is   18:30: 0   18/ 8/94 UTC                                         Records =    7082

COARSE Temperature Calibration: Calib Date  MAR 22,91      Channel=   2

CT0=-0.2480000E+01 CT1= 0.2292700E-01 CT2=-0.1629000E-05 CT3= 0.2100800E-08

Conductivity Calibration:       Calib Date  MAR 22,91      Channel=   3

C0= 0.5500500E+00  C1= 0.4688000E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Pressure Calibrations:          Calib Date  MAY 24,91      Channel=   4

PC0=-0.2975000E+02 PC1= 0.7508600E+00 PC2= 0.0000000E+00 PC3= 0.0000000E+00

Compass Corrections:            Calib Date  MAR 16,91      Channel=   5

25.0 26.0 27.0 27.0 28.0 29.0 29.0 29.0 28.0 28.0 29.0 29.0

29.0 29.0 28.0 28.0 27.0 26.0 25.0 24.0 23.0 22.0 22.0 22.0

21.0 21.0 20.0 20.0 19.0 19.0 19.0 20.0 20.0 20.0 19.0 20.0

Speed Calibrations:             Calib Date A1, LAP-20      Channel=   6

C0=  0.015000     C1= 0.007000     Gear Ratio= 4.0

processing date : 23-AUG-95 09:37:23

Input File Name : E01__0025_940816_A.VIEW_CLIP

Output File Name : E01__0025_940816_A.cvrt

Site: E01      Depth =   25 M Estevan Point (near shoLat 4917.0N    Long 12635.6W   Meter#   3506

Start Time is    4:30: 0   16/ 8/94 UTC                                         Records =     304

COARSE Temperature Calibration: Calib Date  DEC 12,90      Channel=   2

CT0=-0.2477000E+01 CT1= 0.2276100E-01 CT2= 0.1430900E-05 CT3= 0.1961700E-08

Conductivity Calibration:       Calib Date  DEC 12,90      Channel=   3

C0= 0.5683200E+00  C1= 0.4927000E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Pressure Calibrations:          Calib Date  DEC 10,90      Channel=   4

PC0=-0.1042000E+00 PC1= 0.1501400E+00 PC2= 0.0000000E+00 PC3= 0.0000000E+00

Compass Corrections:            Calib Date  MAR 25,91      Channel=   5

21.0 22.0 22.0 22.0 22.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0 23.0

23.0 23.0 22.0 22.0 21.0 21.0 21.0 21.0 21.0 20.0 20.0 20.0

Speed Calibrations:             Calib Date LAP-20,E01      Channel=   6

C0=  0.011000     C1= 0.007817     Gear Ratio= 4.0

processing date :  3-AUG-95 14:11:01

Input File Name : E01__0035_940816_C.VIEW_CLIP

Output File Name : E01__0035_940816_A.cvrt

Site: E01      Depth =   35 M Estevan Point (near shoLat 4917.0N    Long 12635.6W   Meter#   5361

Start Time is    4:30: 0   16/ 8/94 UTC                                         Records =   12228

COARSE Temperature Calibration: Calib Date  JUL 13,90      Channel=   2

CT0=-0.2513000E+01 CT1= 0.2314600E-01 CT2=-0.2104000E-05 CT3= 0.2402800E-08

Conductivity Calibration:       Calib Date  JUL 13,90      Channel=   3

C0= 0.5682300E+00  C1= 0.4921500E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

Pressure Calibrations:          Calib Date  JUL 13,94      Channel=   4

PC0=-0.1047000E+03 PC1= 0.2235400E+01 PC2= 0.0000000E+00 PC3= 0.0000000E+00

Compass Corrections:            Calib Date  Mar 4, 92      Channel=   5

20.0 21.0 21.0 21.0 21.0 22.0 22.0 22.0 21.0 21.0 21.0 21.0

22.0 23.0 23.0 23.0 23.0 23.0 24.0 24.0 24.0 23.0 23.0 23.0

23.0 23.0 22.0 21.0 20.0 20.0 20.0 20.0 20.0 19.0 19.0 19.0

Speed Calibrations:             Calib Date E01,LAP-20      Channel=   6

C0=  0.015000     C1= 0.007000     Gear Ratio= 4.0

processing date : 15-AUG-95 09:08:49

Input File Name : E01__0075_940816_A.VIEW_CLIP

Output File Name : E01__0075_940816_A.cvrt

Site: E01      Depth =   75 M Estevan Point (near shoLat 4917.0N    Long 12635.6W   Meter#   8211

Start Time is    4:30: 0   16/ 8/94 UTC                                         Records =   12406

COARSE Temperature Calibration: Calib Date  MAR 12,92      Channel=   2

CT0=-0.2489000E+01 CT1= 0.2249700E-01 CT2=-0.8374000E-06 CT3= 0.1601400E-08

Conductivity Calibration:       Calib Date  MAR 12,92      Channel=   3

C0= 0.5651300E+00  C1= 0.4951200E-03  C2= 0.0000000E+00  C3= 0.0000000E+00

FINE Temperature Calibration:   Calib Date  MAR 12,92      Channel=   4

XC0= 0.3999400E+01 XC1= 0.7711500E-02 XC2= 0.5316400E-07 XC3= 0.5973800E-10

Compass Corrections:            Calib Date  JUN 19,91      Channel=   5

18.0 18.0 18.0 18.0 19.0 20.0 20.0 20.0 20.0 20.0 20.0 20.0

20.0 20.0 20.0 20.0 20.0 19.0 19.0 18.0 18.0 18.0 17.0 17.0

17.0 17.0 16.0 16.0 15.0 16.0 16.0 16.0 16.0 17.0 15.0 15.0

Speed Calibrations:             Calib Date  E01,LAP20      Channel=   6

C0=  0.011000     C1= 0.007817     Gear Ratio= 4.0

processing date : 22-AUG-95 08:27:31

Input File Name : A1___0102_940818_A.VIEW_CLIP

Output File Name : A1___0102_940818_A.cvrt

Site: A1       Depth =  102 M La Perouse Bank        Lat 4832.1N    Long 12612.5W   Meter#    889

Start Time is   19: 0: 0   18/ 8/94 UTC                                         Records =    4722

channel     depth    sensor       date          T0            T1            T2            T3

2       102      2021    JAN11,93  0.382437E+01  0.772271E-02  0.436818E-08  0.116122E-09

3       107      2021    JAN11,93  0.381995E+01  0.765018E-02  0.510814E-07  0.120171E-09

4       112      2021    JAN11,93  0.382414E+01  0.775382E-02 -0.199954E-06  0.289851E-09

5       117      2021    JAN11,93  0.381980E+01  0.763018E-02  0.694765E-07  0.106244E-09

6       122      2021    JAN11,93  0.381809E+01  0.770693E-02 -0.160402E-07  0.160155E-09

7       127      2021    JAN11,93  0.381533E+01  0.779722E-02 -0.228911E-06  0.288668E-09

8       132      2021    JAN11,93  0.381640E+01  0.775726E-02 -0.224655E-06  0.323570E-09

9       137      2021    JAN11,93  0.381486E+01  0.778523E-02 -0.269165E-06  0.340233E-09

10       142      2021    JAN11,93  0.381995E+01  0.762040E-02  0.111673E-06  0.783384E-10

11       147      2021    JAN11,93  0.382115E+01  0.758850E-02  0.217137E-06  0.102355E-10

12       152      2021    JAN11,93  0.381995E+01  0.765018E-02  0.510814E-07  0.120171E-09

4. Editing after calibration

Used VIEW to edit the salinity files. (*.SAL -> *.VIEW_SAL)

Directions from A1___0175_940818_A.VIEW_SAL had been flipped 180 degrees,

output file is A1___0175_940818_B.VIEW_SAL.

5. Direction average

Direction averaged all current meter files. file names are *.CUR

6. Harmonic Analysis

The results are printed out and stored on files with extensions *.HAR_LOG

7. Standard monthly plot

The standard monthly plots are done on all the files.

--------------------------------------------------------------------------------

This report can be found on CCS$PHYSICS:[AYPLEE.D.LAP20]LAP-20.REPORT

Andrew Lee