Current meter conversion notes

Mar 5, 1998.

Mission  : Deployment (83-10)

Recovery   (83-12)

Date     : May 3, 1983 to Sept 14, 1983.

Project  : NCODE1

Station  : QA1, QB1, QB2, QB3, QC1, QC2, QC3, QC4, QE1, QE2 and QE3

Comments:

There were 31 files converted from CHS format.

--------------------------------------------------------------------------------

** QA1 **

QA1_0055.CUR -- 50m

4245/5 Removed possible fresh water intrusion #5918,5925.

Warning! Avg depth excursion 6.5 dbars, avg max excursion 1.3 dbars!!

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

QA1_0155.CUR -- 155m

5472/8310

WARNING! Large mooring tilts observed, see sn 4245/t5.drav data.

Salinities calculated by the Perkin & Walker formula.

Recovered on cruise IOSOP83-12.

--------------------------------------------------------------------------------

** QB1 **

QB1_0050.CUR -- 50m

731/8310 On time.WARNING! Avg depth excursion 1.6 dbars, avg max excursion

22 dbars!

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

QB1_0150.CUR -- 150m

3694/11 Instrument gimbal bent, shows signs of rubbing on case.

WARNING! Large mooring tilts observed, see sn731/t8310.drav data.

On time.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

--------------------------------------------------------------------------------

** QB2 **

QB2_0080.CUR -- 80m

3699/6 High pressures #620-1100.

WARNING! Avg depth excursion 3.3 dbars, avg max excursion 13.1 dbars.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-10.

QB2_0180.CUR -- 180m

6900/8310 High pressures #780-1200.

WARNING! Large mooring tilts observed, see sn3699/t6.drav data.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

--------------------------------------------------------------------------------

** QB3 **

QB3_0020.CUR -- 20m

3667/7 Release attempts 8,9/9/83, but did not surface. Sighted adrift 3/10/83

and top 3 instruments recovered 21/10/83.High pressures #800-1100

padded out salinity drop #5222-5230.Top 3 instruments recovered but bottom

instr(sn5475/t20.null) lost.See paper file (Oc.Phys)for complete rcvery info.

Salinities calculated by the Perkin & Walker formula.WARNING! Avg depth exc-

ursion 12.2 dbars, avg max excursion 21.4 dbars.

QB3_0115.CUR -- 115m

3666/8 Release attempts 8,9/9/83,but did not surface.Sighted adrift 3/10/83,

recovered 21/10/83.High pressures #800-1100. WARNING! Avg depth excursion 3.4

dbars,avg max excursion 12.1 dbars.Top 3 instr recovered, bottom 2 lost

(sn5475/t20.null,sn5389/t20.null).

Salinities calculated by the Perkin & Walker formula

QB3_0270.CUR -- 270m

3675/6 Release attempts 8,9/9/83, but did not surface.Sighted adrift 3/10/83,

recovered 21/10/83.High pressures #700-1100.Top 3 instr recovered, bottom 2

lost (sn5475/t20.null,sn5389/t20.null). WARNING! Large mooring tilts observed

see sn3666/t8.drav & sn3667/t7.drav data.

Salinities calculated by the Perkin & Walker formula

--------------------------------------------------------------------------------

** QC1 **

QC1_0050.CUR -- 50m

3673/7 Conductivity degrades, #4729 to end, has been padded out, but could be

revived with a little work.

WARNING! Avg depth excursion 1.82 dbars, avg max excursion 11.4 dbars.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

QC1_0150.CUR -- 150m

6917/1   Zero mean noise in cond/salinity channels.Temp & Sal channels padded

#2499-2562.

WARNING! Large mooring tilts observed, see sn3673/t7.drav data.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

QC1_0190.CUR -- 190m

6188/3 ON TIME! Temperature ,conductivity, pressure and direction channels

are garbage and have been padded. Zero speeds have been padded.

-- Kept only the speed channel.

--------------------------------------------------------------------------------

** QC2 **

QC2_0050.CUR -- 50m

3669/6 No pressure signal, so pressure channel padded out.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

QC2_0150.CUR -- 150m

1935/8310 Was deployment '1'

2 records deleted to correct time. No pressure signal so pressure

channel padded out.Number of records obsvd and expected differ but on time

according to time in and out of water.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

QC2_0500.CUR -- 500m

5473/1 # of records obsvd and expected differ but on time according to time

into and out of water. Channel 4 (temp-fine) unuseable as it reads 1023 only.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

--------------------------------------------------------------------------------

** QC3 **

QC3_0020.CUR -- 20m

3705/7 On time.

WARNING! Avg depth excursion 1.7 dbars, avg max excursion 3.89 dbars.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

QC3_0125.CUR -- 125m

2462/1 1 rcrd short,assume start time correct.Instrument & buoy were tangled

on recovery. many zero speeds #6334-6453, zero speeds were padded out.

Pressure port stoppered.Depth (125m) adjusted from nominal target depth (150)

as per mooring tilt data from instr above and below (sn3705/t7 & sn507/t28).

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

QC3_0275.CUR -- 275m

507/28 1st 539 records garbage, so trimmed off and assumed end time correct.

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see sn3705/t7.drav data.

Recovered cruise IOSOP83-12.

QC3_0475.CUR -- 475m

5970/3 On time.

WARNING! Large mooring tilts observed, see sn3705/t7.drav data.

Salinities calculated by the Perkin & Walker formula

Recovered cruise IOSOP83-12.

QC3_0975.CUR -- 975m

5971/2  On time.

Resistor found in bottom of pressure case, so directions may be invalid. Nois

y direction #2592-4500 padded out.  Expanded temp  channel noisy, so use chan

nel 2 temperature.

WARNING! Large mooring tilts observed, see sn3705/t7.drav data.

Salinities calculated by the Perkin & Walker formula

Recovered cruise IOSOP83-12.

--------------------------------------------------------------------------------

** QC4 **

QC4_0180.CUR -- 180m

3789/6  On time.Instr depth changed to 180m from data report (DRCO XX) depth

of 189m due to results from mooring_tilt pgm run on pressure channel.

Salinities calculated by the Perkin & Walker formula

Recovered cruise IOSOP83-12.

QC4_0280.CUR -- 280m

639/8310  On time.

Instr depth changed to 280m from data report (DRCO XX) depth

of 283m due to results from mooring_tilt pgm run on pressure channel.

Salinities calculated by the Perkin & Walker formula

Recovered cruise IOSOP83-12.

QC4_0430.CUR -- 430m

337/8310 On time. Low speeds #3340-4000,#4250-5070. Pressure noisy,sensor may

be faulty. Suspicious cond/sal dropouts removed #3273-3281 & 3402-3412.

Instr depth changed to 430m from data report (DRCO XX) depth

of 429m due to results from mooring_tilt pgm run on pressure channel seen in

upper 2 instruments with more dependable pressure readings(see sn3789/t6.drav

& sn639/t8310.drav data).Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

QC4_0630.CUR -- 630m

5974/2 Large cond/sal jumps #2205-2261,5956-6010 were padded out.

Meter depth (630m) increased from nominal (500m) as per mooring_tilt prgm

results in sn3789/t6.log & sn639/t8310.log (top 2 meters on mooring).

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

QC4_1130.CUR -- 1130m

5471/1 Very slow currents,often below speed threshold,leading to 0 speeds.All

zero speeds were padded.

Meter depth (1130m) increased from nominal (1000m) as per mooring_tilt prgm

results in sn3789/t6.log & sn639/t8310.log (top 2 meters on mooring).

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12.

QC4_2630.CUR -- 2630m

Very slow currents, often below speed threshold, leading to zero speeds.

All zero speeds were padded.

Meter depth (2630m) increased from nominal (2500m) as per mooring_tilt prgm

results in sn3789/t6.log & sn639/t8310.log (top 2 meters on mooring).

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12

--------------------------------------------------------------------------------

** QE1 **

QE1_0025.CUR -- 25m

Mooring shows signs of being hit by fishing tackle, also heavy  bio-fouling

on recovery.

No direction signal, so ch5 padded.

No pressure signal, so ch4 padded.

Speeds slow after record 5486.  Zero speeds padded.

Large cond/sal spikes at records 4106-4113,4298-4417 were padded.

Recovered on cruise IOSOP83-12

Salinities calculated by the Perkin & Walker formula

-- removed pressure and direction channels

--------------------------------------------------------------------------------

** QE2 **

QE2_0052.CUR -- 52m

Out of tape on recovery.

A number of very large cond/sal spikes were padded out, as they looked more

like noise than a fresh water intrusion.

Salinities calculated by the Perkin & Walker formula

QE2_0153.CUR -- 153m

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12

--------------------------------------------------------------------------------

** QE3  **

QE3_0075.CUR -- 75m

1 extra record on translation.

Assume raw record #1 is due to manual trigger.

A number of very large cond/sal spikes were padded out as they looked more

more like noise than a fresh water intrusion.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12

QE3_0178.CUR -- 178m

On Time.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP83-12

--------------------------------------------------------------------------------