Current meter conversion notes

Feb 16, 1998.

Mission  : Deployment (83-12)

Recovery   (84-10, 84-11)

Date     : Sept 9, 1983 to April 19, 1984.

Project  : NCODE2

Station  : QA1, QB1, QB2, QB3, QC1, QC2, QC3, QE1, QE2 and QE3

Comments:

There were 25 files converted from CHS format.

--------------------------------------------------------------------------------

** QA1 **

QA1_0060.CUR -- 60m

4245/6  Release attempt 24/4/84.  Recovered by dragging 12/8/84 Tape ran off

end.  Slight moisture in case.  Instrument gimbal  rod damaged by grapple. AS

SUME ON TIME.

Salinities calculated by the Perkin & Walker formula

WARNING! Avg depth excursion 2.5 dbars, avg max excursion 8.4 dbars.

Recovered using contract vessel "Arrawac".

QA1_0160.CUR -- 160m

4316/1 Release attempted 24/4/84.  Recovered by dragging 12/8/84. Tape ran of

f end.  ASSUME ON TIME.

Salinities calculated by the Perkin & Walker formula

Warning! Large mooring tilts observed, see sn4245/t6.drav data.

Recovered by contract vessel "Arrawac".

-- Salinities seemed a bit too high.

--------------------------------------------------------------------------------

** QB1 **

QB1_0060.CUR -- 60m

3768A/1 Recovered by Pisces. Tape full and bat. dead on recovery.  ASSUME ON

TIME. T,C,P not calib, so used appx calib coeffs   for T&C. Many salinity spi

kes padded out.  many zero speeds     after #12344 were padded out.       1:A

B      6:BE      2:PE

Salinities calculated by the Perkin & Walker formula

Changed instr.depth to 60m (50m nominal) as per sn3694/t12.drav press data.

--Raw pressure

QB1_0160.CUR -- 160m

3694/12   Wire rubbed across bow as mooring deployed. Recovered by Pisces.  T

ape full & batteries dead. ASSUME ON TIME.         Many 0 speeds - all interp

olated or padded.                       13867:SY    593:LR    586:SR      1:A

B    593:BE    593:PE

Salinities calculated by the Perkin & Walker formula

Instr. depth increased to 160m (150m nom) as per mooring_tilt pgm data.

--------------------------------------------------------------------------------

** QB2 **

QB2_0150.CUR -- 150m

1502/1  Mooring hit by Black Cod boat in late July/84, top & bttm instruments

(sn3699/t20.null & sn3014/t20.null) lost, only sn1502 recovered.This instrum-

ent returned 5 Sept/84.Tape full and battery dead. Assume on time after del-

eting 1 rcd. Cond and press channels uncalibrated.

--No salinity

--------------------------------------------------------------------------------

** QB3 **

QB3_0000.CUR -- 0m

3667/8. Instrument flooded. Tape salvaged, but short (4700 recs). Top buoy lo

st, meter slid down to 2nd buoy. ASSUME ON TIME.   Meter on surface at start.

*******Direction is garbage********, speed and pressure are highly questiona

ble.

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see sn3675/t7.drav & sn3666/t9.drav

data.

Recovered on cruise IOSOP84-11.

QB3_0085.CUR -- 85m

3675/7. ON TIME. Direction noisy to #3500, possibly due to fouling by top wir

e (signs of wire rubbing on casing). Sudden speed  drop #3536, but increases

to normal afterwards. Mooring depressed to 266m Apr 13-22.

Salinities calculated by the Perkin & Walker formula

WARNING! Avg depth excursion 10 dbars, avg max excursion 52 dbars.

Recovered on cruise IOSOP84-11.

QB3_0235.CUR -- 235m

3666/9. ON TIME.  Signs of wire rubbing on instrument casing.   Meter 65 m

shallower than nominal.

Salinities calculated by the Perkin & Walker formula.

WARNING! Avg depth excursion 10 dbars, avg max excursion 40 dbars.

Recovered on cruise IOSOP84-11.

QB3_0435.CUR -- 435m

5972/4. ON TIME.  Signs of wire rubbing on instrument casing.   Meter 65 m

shallower than nominal depth as per sn3675/t7 & sn3666/t9 mooring_tilt pgm

data.

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see sn3675/t7.drav & sn3666/t9.drav

data.

Recovered on cruise IOSOP84-11.

--------------------------------------------------------------------------------

** QC1 **

QC1_0200.CUR -- 200m

3673/8  Mooring line cut above bottom buoy, lost top 2 meters, sn6188/t4.null

and sn1501/t20.null.

On time.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP84-11.

--------------------------------------------------------------------------------

** QC2 **

QC2_0065.CUR -- 65m

3669/7. ON TIME.

Salinities calculated by the Perkin & Walker formula.

WARNING! Avg depth excursion 4.0 dbars, avg max excursion 24 dbars.Instrument

depth increased to 65m  from nom as per mooring_tilt pgm data.

Recovered on cruise IOSOP84-11.

QC2_0165.CUR -- 165m

3789/7. ON TIME.

Salinities calculated by the Perkin & Walker formula

WARNING! Avg depth excursion 3.7 dbars, avg max excursion 21.7 dbars.Instr

depth increased to 165m from nom as per mooring_tilt pgm data.

Recovered on cruise IOSOP84-11.

QC2_0315.CUR -- 315m

2361/1. ON TIME. Cond/sal high,due to poor conductivity calibration.

Instr depth increased to 315m from nom as per mooring_tilt pgm data from inst

sn3669/t7.drav and sn3789/t7.drav.

Salinities calculated by the Perkin & Walker formula.

WARNING! Large mooring tilts observed, see mooring_tilt data noted above.

Recovered on cruise IOSOP84-11.

QC2_0515.CUR -- 515m

5973/4  ON TIME. Padded out cond/sal jump #1563-1688. Field notes indicate

compass may not have been checked prior to deployment, use caution.Instrument

depth incr to 515m as per mooring_tilt data on top 2 instruments, see sn3669/

t7.drav and sn3789/t7.drav data.

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see mooring_tilt pgm data from files

mentioned above.

Recovered on cruise IOSOP84-11.

--------------------------------------------------------------------------------

** QC3  **

QC3_0070.CUR -- 70m

3705/8  ON TIME.  Salinity becomes low near end, also strange   looking cond

signal, so sensor may be faulty.  Countless press. spikes removed throughout

data set.

Salinities calculated by the Perkin & Walker formula

WARNING! Avg depth excursion 5.2 dbars, avg max excursion 9.6 dbars.

Instr depth incr to 70m as per mooring_tilt pgm data.

Recovered on cruise IOSOP84-11.

QC3_0170.CUR -- 170m

507/29. ON TIME.

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see sn3705/t8.drav data.Instr depth

incr to 170m as per mooring_tilt pgm data.

Recovered on cruise IOSOP84-11.

QC3_0320.CUR -- 320m

1500/1. ON TIME.Not calib'd @IOS,got calibs from Aanderaa over phone.Increas-

ed depth from nominal as per upper 2 instruments (sn3705/t8 & sn507/t29)

mooring_tilt pgm data.

WARNING! Large mooring tilts observed, see sn3705/t8.drav data.

Salinities calculated by the Perkin & Walker formula.

Recovered on cruise IOSOP84-11.

QC3_0520.CUR -- 520m

5970/4. ON TIME.  Salinity low from seq# 8256-8885, but not    padded out, as

couldn't decide whether real or not.

Instr. depth increased 20m from nom.as per pressure data from top 2 instrum-

ents (sn3705/t8,sn507/t29).

Salinities calculated by the Perkin & Walker formula.

WARNING! Large mooring tilts observed, see sn3705/t8.drav data.

Recovered on cruise IOSOP84-11.

QC3_1020.CUR -- 1020m

5971/3. ON TIME.  Many zero speeds, due to low current. All zero speeds were

padded.Inst.depth increased by 20m as per pressure data from top 2 instrument

s (sn3705/t8 & sn507/t29).WARNING!Large mooring tilts observed, see sn3705/t8

data.

Salinities calculated by the Perkin & Walker formula.

Recovered on cruise IOSOP84-11.

--------------------------------------------------------------------------------

** QE1 **

QE1_0038.CUR  -- 38m

739/2 Mooring heavily fouled, but sensor head less fouled.ON TIME. Direction

extremely noisy - cannot be fully despiked.

Channel 4 (pressure port plugged) only reads 1023.

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP84-11.

--------------------------------------------------------------------------------

** QE2 **

QE2_0050.CUR -- 50m

3688/9  ON TIME.  Water gradually becomes quite fresh over last month.

Salinities calculated by the Perkin & Walker formula

WARNING! Avg depth excursion 2.0 dbars, avg max excursion 8.5 dbars.

Recovered on cruise IOSOP84-11.

QE2_0150.CUR -- 150m

736/26  ON TIME.

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see sn3688/t9.drav data.

Recovered on cruise IOSOP84-11.

--------------------------------------------------------------------------------

** QE3 **

QE3_0070.CUR -- 70m

3684/13 Mooring caught by fish boat 13/1/84.  ON TIME.

Salinities calculated by the Perkin & Walker formula

WARNING! Avg depth excursion 2.5 dbars, avg max excursion 11.1 dbars.

Recovered on cruise IOSOP84-10.

QE3_0170.CUR -- 170m

314/31 Mooring hit by fish boat 13/1/84.  Compass replaced before deployment

(#5157) as faulty. No compass calib'n done. On Time.

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see sn3684/t13.drav data.

Recovered on cruise IOSOP84-10.

QE3_0320.CUR -- 320m

5974/3  Mooring caught by fisherman 13/1/84.  ON TIME.          Speeds go to

0 on 28/12/83 (seq# 5051).  Direction is highly    suspect, as never changes

very much.Salinities calculated by the Perkin & Walker formula.

WARNING! Large mooring tilts observed, see sn3684/t13.drav data.Instr depth

increased to 320m from nom as per mooring_tilt pgm data from sn3684/t13 and

sn314/t31.

Recovered on cruise IOSOP84-10.

--------------------------------------------------------------------------------