Current meter conversion notes

Feb 16, 1998.

Mission  : Deployment (84-10, 84-11)

Recovery   (85-10, 85-14)

Date     : Oct 17, 1984 - July 4, 1985.

Project  : NCODE4

Station  : QE1, QE2, QE3, QE4, QE5, QE6, QF1, QF2, QF3 and QF4

Comments:

There were 27 files converted from CHS format.

--------------------------------------------------------------------------------

** QE1 **

QE1_0025.CUR -- 25m

3768a/8  Clapper failed on encoder. 119 records to many. Trimmed to #8550 as

to many translation errors after here.  Assume on  time.  Sal,Temp,Cond and d

irection failed at #180, speed only to the end.  Used appx temp & cond calibr

ations.Channel 4 (pressure) uncalibrated data.

Salinities calculated by the Perkin & Walker formula.

Designation of inst as "3768a" done by Aanderaa Instruments-this "a" designat

ion is not for the tape, instrument only (pers.comm'n w John Love,OP 04-I-93)

Recovered on cruise IOSOP85-10.

--------------------------------------------------------------------------------

** QE2 **

QE2_0170.CUR -- 170m

1500/2  Top instrument & buoy cut off, instrument fell to bottom at #6923 (22

:30 10/3/85).  On time after changing start time   and deleting 14 records.

Used unexpanded cond calibration.       12324:SY     18:LR     28:SR      1:A

B     53:BE      3:PE

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP85-10.

--------------------------------------------------------------------------------

** QE3 **

QE3_0160.CUR -- 160m

4294/2  Caught by fisherman 21:00 14/4/85.  Handled roughly, no endplate on p

ressure case - assume occured on deck as no internal corrosion.  No end time

as batteries dead. Zero speeds #6294-6753 padded out.

Depth excursion pgm run and noted minimum depth of 159 m.

Salinities calculated by the Perkin & Walker formula.

QE3__0310.CUR -- 310m

3684/2  Caught by fisherman 21:00 14/4/85.  Handled roughly on  deck. Zero sp

eeds 2929-2937 padded out. Many 0 speeds 5000-8000,not padded out as speed wa

s low. Cond/Sal spike 7699-7701 padded out.Depth excursion pgm run(see

t2.log) and noted minimum depth of 309 m.

Salinities calculated by the Perkin & Walker formula

--------------------------------------------------------------------------------

** QE4 **

QE4_0060.CUR -- 60m

3679/2  On Time. T & C drop .5 deg & .5 mmho/cm at #3434, but   pressure rema

ins same, so assume real drop. Large cond/sal drops at 11127-11158,11976-1199

2,12002-12036,12052-12074,12194-12225,12239-12263 were padded out.

Depth excursion pgm run and noted min inst depth of 57 m.

Salinities calculated by the Perkin & Walker formula

Recovered cruise IOSOP85-10.

QE4_0160.CUR -- 160m

3688/11  Correct # of records, but deleted 1 as appeared to be  a translation

error.  This makes it 1 record short, but the time out of water is correct.

Assume on time.                        12307:SY     17:LR      2:SR      0:A

B     21:BE     32:PE

Salinities calculated by the Perkin & Walker formula

Depth excursion pgm run and noted min inst depth of 159 m.

Recovered on cruise IOSOP85-10.

--------------------------------------------------------------------------------

** QE5 **

QE5_0050.CUR -- 50m

3694/1  Tape wound on capstan, only 1033 records.  Assume on    time.  Many 0

speeds, interpolated where possible, rest padded.

931:SY    102:LR    102:SR      0:A

B    113:BE    108:PE

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP85-10.

QE5_0150.CUR -- 150m

1502/2 Poor batt conn to motor,probably not working on recovery. Missing 364

recs, assume they were lost at end, and on time.   Cond looks bad (every othe

r cond is 1023).  Temp is very erratic,suspect it may be bad.Channel 3 & 4

raw data.

Salinities calculated by the Perkin & Walker formula

Recovered cruise IOSOP85-10.

QE5_0300.CUR -- 300m

2361/2  on time.Zero speeds #6756-7507. All consecutive 0 speeds padded

using  Zpad. Cond/Sal drop 830-864 left in.

12331:SY      0:LR      0:SR      0:A

B      0:BE      0:PE

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP85-10.

--------------------------------------------------------------------------------

** QE6 **

QE6_0050.CUR -- 50m

3695/2  On Time.WARNING! Avg depth excursion 2.51 dbars, avg max excursion

10.7 dbars.

12336:SY      0:LR      0:SR      0:A

B      0:BE      0:PE

Salinities calculated by the Perkin & Walker formula

Recovered cruise IOSOP85-10.

QE6_0150.CUR -- 150m

3789/9  On Time. WARNING! Avg depth excursion 2.36 dbars, avg max excursions

of 10.0 dbars.

12337:SY      0:LR      0:SR      0:A

B      0:BE      0:PE

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP85-10.

QE6_0300.CUR -- 300m

7337/1 Assume on time after deleting 7 records, this leaves the file 1 rec sh

ort, but assume it is lost at the end.             Used poor cond calibration

. Zero speed 3063-3089. many low and  zero speeds after 7000. All consecutive

0 speeds padded out.

Salinities calculated by the Perkin & Walker formula

.WARNING! Large mooring tilts observed,see sn3695/t2.drav and sn3789/t9.drav

data.

Recovered cruise IOSOP85-10.

QE6_0500.CUR -- 500m

3387/2  On Time. Clock 9 min slow on recovery.                  Used poor con

d calibration. Padded 0 speeds 9206-9223,          12082-12103,12195-12202. T

emp & Xtemp are very noisy. Use Temp  for comp sal. As many large temp spikes

were removed as poss.

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see sn3695/t2.drav and

sn3789/t9.drav data.

Recovered on cruise IOSOP85-10.

QE6_0750.CUR -- 750m

5972/6  On Time. Many 0 speeds throughout record. After 8334    almost all th

e speeds were 0, so padded out 8334-end.           All consecutive 0 speeds p

added. Cond/sal drops padded out seq#s1660-1676,1709-1720,4322-4527.

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see sn3695/t2.drav and sn3789/t9.

drav data.

Recovered on cruise IOSOP85-10.

--------------------------------------------------------------------------------

** QF1 **

QF1_0050.CUR -- 50m

3673/12  Last record 6 minutes early.  On time!

WARNING! Avg depth excursion 7.32 dbars, avg max excursion 59.0 dbars.

12424:SY      1:LR      1:SR      0:A

B      2:BE      2:PE

Salinities calculated by the Perkin & Walker formula

Recovered cruise IOSOP85-10.

QF1_0150.CUR -- 150m

3669/9  last record 1 min early. On time.Cond/sal noisy in spots (cond bit

wobble).

12394:SY      0:LR      0:SR      9:A

B      0:BE      0:PE

Salinities calculated by the Perkin & Walker formula.

WARNING! Avg depth excursion 6.5 dbars, avg max excursion 46.3 dbars.

Recovered on cruise IOSOP85-10.

QF1_0350.CUR -- 350m

1702/11 Let-out spool out of round, rubbing on pressure sensor. 7 records sho

rt, Assume records lost at end and meter on time.  Used approximate cond cali

bration. Pressure sensor plugged. Cond/Sal spike #11457-#11503 padded out.

.

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see sn3673/t12.drav and sn3669/t9.drav

data.

Recovered on cruise IOSOP85-10.

-- Pressure channel was removed.

--------------------------------------------------------------------------------

** QF2 **

QF2_0055.CUR -- 55m

6101/11  mooring didn`t release, recovered by dragging 7/12/84. out of tape.

Assume on time after del 1 rec. Alternating time/pr word in ch4. Time word ch

ecks out ok.  Every other pressure is interpolated.  Meter seems to have sunk

at #1641, trim to here.

Salinities calculated by the Perkin & Walker formula

WARNING! Avg depth excursion 13 dbars,avg max excursion 96.0 dbars.

Recovered cruise IOSOP85-14.

QF2_0155.CUR -- 155m

6102/2 Mooring didn't release,recovered by dragging. Out of tape. Assume on t

ime after del 3 recs. Alternating time/pres word.  Time word checks out ok. E

very other pres interpolated. Dir is  fouled seq# 2970-4420,4885-5035. Padded

these records out.Salinities calculated by the Perkin & Walker formula.

WARNING! Avg depth excursion 48.5 dbars, avg max excursion 155.5 dbars.

Mooring recovered cruise IOSOP85-14.

QF2_0405.CUR -- 405m

1704/11  Mooring didn't release, recovered by dragging 7/12/84. Out of tape.

Assume on time!  Direction possible fouled #5650- #5695, but record not padd

ed out. Used approximate cond calib. 12500:SY 0:LR 0:S 3:A

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see sn6101/t11.drav and sn6102/t2.drav

data. Inst.depth incr by 5m as upper 2 inst (sn6101 & sn6102) showed 5m incr.

Recovered cruise IOSOP85-14.

-- Pressure channel was removed.

--------------------------------------------------------------------------------

** QF3 **

QF3_0050.CUR -- 50m

3665/2  Mooring did not release, recovered by dragging 8/12/85. Out of tape.

Assume on time. Speed 0 throughout record.

12982:SY      5:LR      0:SR      0:A

B     10:BE      0:PE

CAUTION -- Data quality not assigned

-- Speed channel was removed.

QF3_0150.CUR -- 150m

3698/2  Mooring didn't release, recovered by dragging 8/12/85.  Out of tape.

Assume on time!

13260:SY      0:LR      1:SR      2:A

B     50:BE     30:PE

Salinities calculated by the Perkin & Walker formula

Recovered on cruise IOSOP85-14.

--------------------------------------------------------------------------------

** QF4 **

QF4_0060.CUR -- 60m

3675/9  Tape ran out, assume on time.Speeds slow near end, alot of zero

re       3:A

QF4_0160.CUR -- 160m

3666/15  Tape ran out, assume on  time.                         Many 0 speeds

from #7705-end.  All 0 speeds padded out.Cond dropout #4144-4152, 6965-6967

removed.                       11869:SY      0:LR      1:SR      1:A

B 1:BE 2:PE.Salinities calculated by the Perkin & Walker formula.

Note:Inst file depths of instruments above and below this meter have been

changed (see sn3675/t9 and sn3705/t10) so this instr.depth changed to 160m.

WARNING! Avg depth excursion 3.42 dbars, avg max excursion 15.8 dbars.

Recovered on cruise IOSOP85-10.

QF4_0310.CUR -- 310m

3705/10  On time, after deleting 1 record.  Clock 4 min fast on recovery.  Co

nd/Sal dropout 10480-10494 padded out.

12377:SY      0:LR      3:SR      2:A

B      1:BE      0:PE

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see sn3675/t9.drav and sn3666/t15.drav

data.Changed instr. depth as per mooring_tilt data.

Recovered cruise IOSOP85-10.

QF4_0510.CUR -- 510m

5973/5  Tape ran out, assume on time.  Clock 3 min slow on rec. Used poor con

d calibration.  Direction bad after #11385.  STCX  bad after #11862, bad reco

rds padded out.                         11912:SY    381:LR     22:SR      0:A

B     65:BE      5:PE

Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see rcm4/sn3675/t9.drav and

rcm4/sn3666/t15.drav data.Incr sn5973 depth to 510 m as per this data above.

Recovered on cruise IOSOP85-10.

QF4_0760.CUR -- 760m

5974/5  Tape ran out, assume on time.Clock 4min:20sec fast on   recovery. Zer

o speeds 7935-9380 & 10347-end were padded out.    Strange unidirectional cur

rents after 2877.                       11591:SY      3:LR      9:SR      0:A

B     30:BE     10:PE Salinities calculated by the Perkin & Walker formula

WARNING! Large mooring tilts observed, see rcm4/sn3675/t9.drav and rcm4/sn

3666/t15.drav data.Instr depth increased to 760m as upper instruments on

mooring showed a 10m depth shift from nominal.

Recovered on cruise IOSOP85-10.

---------------------------------------------------------------------------------