Program S4rot applies rotation correction to the data

collected from the S4 current meter. The input file, in

ASCII, is created from program APPUBC, with the options F2

(Output) set to  Header + Data,  F4 (OutPut Device) set to

File and F5 (Current as speed/dir) set to No, in  the

OutPut Menu.

The program can be run in either of the following

two ways:

1. Type S4Rot and respond to the following

prompts:

Input File Name:

Output File Name (inputfile.out):

Rotation (deg):

2. Type all information on a single command line, using the

following syntax:

S4Rot [inputfile ]outputfile Rnnn

Argument		Description

------------------------------------------------------------------------

[inputfile     	File containing input data. "[" designates it as

input.

]outputfile	File to where the output is  directed to. "]"

designates it as output. This is an  optional

argument, the default output file is named

inputfile.out.

Rnnn    	Rotation value in deg.

See below examples.

_______________________________________________________________________

Examples:

1) S4Rot [Inlet.S4A R22

Program runs with input file Inlet.S4A, and puts results in

Inlet.out, the default file and applies a rotation of 22 degs.

2) S4Rot [Inlet.S4A ]InletCal.out R15

Program uses Inlet.S4A and InletCal.out as input and output files

respectively. Inst12 is the calibration file.

3) for %f in (*.s4a) do S4Rot [%f R22

The above statement is way to process all files that match

*.s4a and apply a rotation of 22 degs.

For more detail see BATCH PROCESSING section in the

DOS manual.