Current meter processing notes

Revised Nov 2, 2001

Mission : 9935 (deployment), 0006 (recovery)

Date    : Oct 3, 1999 - May 17, 2000

Project : Wide Straits 4

Station : JF1A, JF2A, JF2B, JF2D, JF2E, and JF3A.

Comments: There were 18 current meter files processed.

JF1A: Mid-channel, west side

JF1A_015     15m. OK

(S4, 05451466)

JF1A_030     30m. DSU was full . Data after 2300 utc May 10,2000 were not

(10320/11)        recorded. Clock was 6 minutes behind.

There were 28 records missing at the beginning.

JF1A_090     90m. Instrument stopped recording after 0230 utc Dec 17, 1999.

(8819/3)          It started recording again after the anchor was released.

JF1A_145    145m. Last record was at 1700 utc May 4, 2000.

(6506/24)         Assumed on time from the beginning to April 16.

However, records could be missing after April 16.

Despiked data after April 16, 2000.

JF2A: Canadian side, near shore

Mooring was 3m shallower than nominal depth.

JF2A_027     27m. The clock was 1 minute behind.

(737/7)           The instrument was cut off by fish boat after

2000 utc April 20, 2000, recovered in Kyquot Sound.

The averaged depth was 27m, 3m shallower

than nominal depth.

JF2A_087     87m. The clock was 1 minute ans 42 seconds ahead.

(7337/27)         The averaged depth of the upper meter was

3m shallower than nominal depth.

The depth of this meter was corrected to 87m accordingly.

JF2B:  Canadian side, off shore

JF2B_029     29m. DSU was full, last record was at 2300 May 9, 2000.

(9049/4938)       Assumed on time.

Padded salinity values after 0800 May 4, 2000.

JF2B_089     89m. Some wrapped around in raw speeds.

(1935/20)

JF2B_144    144m. There were 3 extra records in the original raw data.

(7897/5)          Two records were deleted between Oct 2 and May 17.

The 1998 calibration coefficents were used to calibrate

the data

JF2D: American side, off shore

The mooring was moved to a shallower position

after 1030 utc March 30, 2000.

JF2D_031     31m. DSU(5989) was full, last record was at 1830 May 10, 2000.

(718/1)           One record was missing in Dec 25, 1999.

One record was added at 2330 utc Dec 25, 1999.

JF2D_091     91m. There was one extra record. It seemed that the clock

(1604/11)         was 31 minutes and 4 seconds slow.

Rotor was damaged after 1030 utc March 30, 2000.

JF2D_146    146m. OK

(3228/5)

JF2E: American side, near shore

This mooring was deployed 5m shallower than nominal depth. The mooring

was hit twice (Oct 25 and 30)

JF2E_025     25m. The clock was 9 minutes and 3 seconds ahead.

(4075/9)          Temperature channel was removed. No salinity values.

Speed spikes were edited by diving the raw values by 4

if the values greater than 500.

Averaged pressure was 25m and the depth of the instrument

was corrected accordingly.

JF2E_085     85m. The clock was 25 minutes and 24 seconds ahead.

(2525/22)         The depth of this instrument was corrected to 85m.

JF3A: Mid channel, east side

JF3A_017     17m. Calibrated data by running APP273 then refomatted data

(S4,05451467)      by running REFORMAT. Data were then converted to IOS

format.

Padded all channels from 1700 utc Nov 6 to

0815 utc Nov 9, 1999.

JF3A_032     32m. DSU was full or locked up.

(10698/4)         Records were missing from 0500 utc Oct 1 to

1530 utc Oct 5, 99.

Also records were missing after 2030 utc April 22, 2000.

The clock was 6 minutes behind.

Expanded temperature values were overrange except

between Jan 9 and Feb 22, 2000

JF3A_092     92m. The clock was 1 minute and 30 seconds ahead.

(8830/9)          Very noisy temperature and conductivity after Dec 2, 1999.

JF3A_147    147m. The clock was 4 minutes ahead.

(3501/14)         Averaged pressure was 132m, 15m shallower than

nominal depth.

1.  Convert raw data files

Using IOSSHELL to add headers to raw data files.

2.  Clip data

Using IOSSHELL to clip raw data.

3.  Editing

Used program VIEW to edit.

4.  Calibration

Used program in IOSSHELL to calibrate data.  Coefficients are stored on CCF

files and also in the headers.

5. Editing after calibration

Used program VIEW to edit.

6. Direction average (current meter data only)

STATION    STARTING          RECORDS  DEPTH(M)  INTRVAL   POS        CHANS

TAPE ID    ENDING TIME(utc)

---------------------------------------------------------------------------

JF1A       1700 Oct 03, 99  21801     15        15        48 23.08N  RTrPV

05451466   1900 May 17, 00                               124 20.35W

JF1A       1730 Oct 03, 99  10572     30        30        48 23.08N  RTrPV

10320/11   2300 May 10, 00                               124 20.35W

JF1A       1800 Oct 03, 99   3444     90        30        48 23.08N  RTrPV

8819/3     1130 Dec 14, 99                               124 20.35W

JF1A       1800 Oct 03, 99  10270    145        30        48 23.08N  RTrXV

6506/24    1630 May 04, 00                               124 20.35W

JF2A       1730 Oct 02, 99   9557     27        30        48 24.79N  RTrPV

737/7      1930 Apr 18, 00                               124 09.94W

JF2A       1730 Oct 02, 99  10938     87        30        48 24.79N  RTrXV

7337/27    1400 May 17, 00                               124 09.94W

JF2B       1800 Oct 02, 99  10571     29        30        48 22.97N  RTrPV

9049/4938  2300 May 09, 00                               124 11.05W

JF2B       1830 Oct 02, 99  10941     89        30        48 22.97N  RTrXV

1935/20    1630 May 17, 00                               124 11.05W

JF2B       1830 Oct 02, 99  10941    144        30        48 22.97N  RTrXV

7897/5     1630 May 17, 00                               124 11.02W

JF2D       2300 Oct 02, 99   8616     31        30        48 19.10N  RTrPV

718/1      1030 Mar 30, 00                               124 12.94W

JF2D       2330 Oct 02, 99   8614     91        30        48 19.10N  RTrXV

1604/11    1000 Mar 30, 00                               124 12.94W

JF2D       2330 Oct 02, 99   8614    146        30        48 19.10N  RTrXV

3228/5     1000 Mar 30, 00                               124 12.94W

JF2E       0100 Oct 03, 99  10793     25        30        48 15.09N  RTrPV

4075/9     2100 May 14, 00                               124 09.80W

JF2E       0100 Oct 03, 99  10793     85        30        48 15.09N  RTrXV

2525/22    2100 May 14, 00                               124 09.80W

JF3A       1945 Oct 03, 99  21497     17        15        48 17.09N  RTrPV

05451467   1745 May 14, 00                               123 58.69W

JF3A       1600 Oct 05, 99   9598     32        30        48 17.09N  RTrXV

10698/4    1430 Apr 22, 00                               123 58.69W

JF3A       2100 Oct 03, 99  10480     92        30        48 17.09N  RTrPV

8830/9     0430 May 09, 00                               123 58.69W

JF3A       2100 Oct 03, 99  10745    147        30        48 17.09N  RTrPV

3501/14    1700 May 14, 00                               123 58.69W

7. Moving average

Moving average done by IOSSHELL (*.CUR > *.F3P)

8. Harmonic Analysis

By using harmonic programs in IOSSHELL

9. Standard monthly plot

Plotted.

--------------------------------------------------------------------------------

Andrew Lee